Application Note for T8pBurstEx

# General

T8pBurstEx is a simple example console application demonstrating the use of the DMA Direct Sending and Receiving with the OTX Driver using the board's Burst device to access the data using DMA. This application can be used with the following boards:

- Thor-8-PCI-Plus (OTX_DEVICE_THOR_8_PCI_PLUS)

- Sleipnir-1-PCI (OTX_DEVICE_SLEIPNIR_1_PCI)

The demo application provides a simple console interface which utilizes simple keystrokes as commands.

Eight T1 or E1 spans are configured. Theses spans can be connected to another T1 or E1 device, or looped with an external loop cable.

The direct sends and receives operate on a highway basis and are hosted by the Burst (OTX_DEVICE_BURST) device.

Pipes are created between the Line interface and the burst device. The pipes act as a logical conduit for the data transfer. The burst device works with highways that have a capacity (speed) that will vary between different types of OTX boards:

Thor-8-PCI-Plus: 	Two 8192 kbps (128 timeslots) bidirectional highways

Sleipnir-1-PCI:		One 2048 kpbs (32 timeslots) bidirectional highway

For Thor-8-PCI-Plus, 2 8192 kbps bidirectional highways are available to handle all of the timeslots for each of the eight line interface. The following Li <-> Burst device connections are made in transmit and receive direction:

| Li | Burst highway | Burst highway timeslots |
| --- | --- | --- |
| 0 | 0 | TS0 – TS31 |
| 1 | 0 | TS32 – TS63 |
| 2 | 0 | TS64 – TS95 |
| 3 | 0 | TS96 – TS127 |
| 4 | 1 | TS0 – TS31 |
| 5 | 1 | TS32 – TS63 |
| 6 | 1 | TS64 – TS95 |
| 7 | 1 | TS96 – TS127 |

For Sleipnir-1-PCI, one 2048 kbps bidirectional highways are available to handle all of the timeslots for each of the eight line interface. The following Li <-> Burst device connections are made in transmit and receive direction:

| Li | Burst highway | Burst highway timeslots |
| --- | --- | --- |
| 0 | 0 | TS0 – TS31 |

OtxBurstReadData() is called from a thread function called DemoBurstReceiver(). This thread handles 4 Lis (either Li0-Li3, or Li4-Li7). Thus there will be two instances of this thread (one for Li0-Li3 and one for Li4-Li7). The thread is started with the ‘r’ or ‘R’ command.

- Starting with Otx Version 2.0, you may now recover a Frame Reference ID and Time from the Burst Device, which can be used for time-stamping frames.  The first call to the Burst device to fill the buffer now is made using the call OtxBurstReadDataEx(), which saves of the frame reference data (which is relevant to the first frame in the returned buffer), then continues to call OtxBurstReadData() to finish filling the local buffer with as much data as is available or can be held.  The frame reference data is then displayed if the verbose setting is active.

OtxBurstSendData() is called from a thread function called DemoBurstSender(). This thread handles 4 Lis (either Li0-Li3, or Li4-Li7). Thus there will be two instances of this thread (one for Li0-Li3 and one for Li4-Li7). The thread is started with the ‘s’ or ‘S’ command.

Starting with Otx Version 2.0, the Burst device can be used in either the existing polling method, or using the new event driver method.  The following outlines the usage of the event driven method (reference the demo T8pBurstEx.c for details).

- Using two new attributes, OTX_ATTR_BURST_RECEIVER_FIFO_TRIGGER and OTX_ATTR_BURST_SENDER_FIFO_TRIGGER, the event signal trigger levels are set to a given amount.  The following example sets the receiver trigger to 16k, thus, enabling an event, OTX_BURST_RECEIVER_EC_DATA_AVAILABLE, to be sent when 16k of data has been received (and will subsequently fire every 16k afterwards).  Conversely, the sender will trigger when 32k of space is available to the application to buffer more data to send with the event OTX_BURST_SENDER_EC_BUF_DEPLETED.

result = OtxDrvSetAttributeValueUint32(hBurst, OTX_ATTR_BURST_RECEIVER_FIFO_TRIGGER,  0x4000);

result = OtxDrvSetAttributeValueUint32(hBurst, OTX_ATTR_BURST_SENDER_FIFO_TRIGGER,    0x8000);

- In lieu of threads that sleep and poll Burst (via OtxBurstReadData[Ex] and OtxBurstSendData[Ex]), the application can wait on the new Burst events using the standard OtxDrvWaitForSingleEvent()API call, and these events will be visible in the event queue pass when the Burst device was opened with a call to the API OtxDrvOpenPhysicalDevice().  When these events are received, the application can process as previously, using calls to OtxBurstReadData() and OtxBurstSendData().  The only change has been how the application handles the data flow, using either a polling or event driven methodology.

## Clock Source

The T8pBurstEx application uses the OTX_CLOCK_SOURCE_INTERNAL clock source, meaning that the board is clock source for the T1 span. The clock source can be changed by modifying the OtxBrdSetClocks() call in the function main(). E.g if to recover the clock from Li0 change the OtxBrdSetClocks() call to:

OtxBrdSetClocks(

g_hPhysDevs.hThorController0,

OTX_CLOCK_MODE_LOCAL,

OTX_CLOCK_SOURCE_LOCAL_0

);

## T1/E1 Configuration

The T1/E1 line configuration is set in the DemoInitT1E1ConfigData() function.

In E1 mode the application configures the line interface in CRC4 Multiframe/HBD3 format.

In T1 mode the application configures the T1 line interface for ESF/B8ZS frame format.

The configuration can easily be modified to match any possible T1/E1 configuration by editing the settings of the struct members in the DemoInitT1E1ConfigData() function.

# Usage Examples

## T1 Usage Example for Thor-8-PCI-Plus

To run T8pBurstEx in T1 mode with reception of all 24 timeslots plus the F-bits and transmission on 24 timeslots (the line interface will automatically generate the outgoing F-bits), the following command line parameters could be used:

T8pBurstEx /mt /n1600 /b0           (for board 0)

T8pBurstEx /mt /n1600 /b1           (for board 1)

To start receiving data (no file logging) from Li0-Li3

press ‘0’

press ‘r’

To start receiving data (no file logging) from Li4-Li7

press ‘4’

press ‘r’

To start sending a repeated buffer from Li0-Li3

press ‘0’

press ‘s’

To start sending a repeated buffer from Li4-Li7

press ‘4’

press ‘s’

- Starting with Otx version 2.0, the T8pBurstEx demo has implemented on-the-fly enabling/disabling of the Burst device (using keyboard commands ‘e’ for Enable and ‘d’ for Disable).  With the new methodology of event driven Burst data flow, it is now easier to control the states of the buffers, thus allowing for tighter control of enabling/disabling the Burst device.  Therefore, before the device can actually send and/or receive data, the command ‘e’ must be issued.  The demo will, then, behave accordingly.  A nice feature of this can be the sending/receiving of a file, the application gains tighter control of the data sent/received.

The following buffer is sent from respective Li:

| Li | Send buffer |
| --- | --- |
| 0 | 0x00 – 0x09 |
| 1 | 0x10 – 0x19 |
| 2 | 0x20 – 0x29 |
| 3 | 0x30 – 0x39 |
| 4 | 0x40 – 0x49 |
| 5 | 0x50 – 0x59 |
| 6 | 0x60 – 0x69 |
| 7 | 0x70 – 0x79 |

To view samples of the received data from any of the Li’s

press ‘0’ 		(‘0’ for Li0, ‘1’ for Li1, etc)

press ‘v’ 		(to toggle verbose mode)

The ‘v’ command toggles verbose mode between on and off.

Although the received data is not written to a file in this example, the continuous stream of data is read from the OTX driver by consecutive calls to OtxBurstReadData(). The data buffer that is returned by OtxBurstReadData() is formatted modulo 128 bytes in the follow fashion:

| Byte Index | Byte description |
| --- | --- |
| 0 | Li0/4:FS/DL bit in LSB bit |
| 1 | Li0/4:TS1 |
| 2 | Li0/4:TS2 |
| .. |  |
| 24 | Li0/4:TS24 |
| 25 | N/A in T1 mode |
| .. |  |
| 31 | N/A in T1 mode |
| 32 | Li1/5: FS/DL bit in LSB bit |
| 33 | Li1/5:TS1 |
| 34 | Li1/5:TS2 |
| .. |  |
| 56 | Li1/5:TS24 |
| 57 | N/A in T1 mode |
| .. |  |
| 63 | N/A in T1 mode |
| 64 | Li2/6: FS/DL bit in LSB bit |
| 65 | Li2/6:TS1 |
| .. |  |
| 96 | Li3/7: FS/DL bit in LSB bit |
| 97 | Li3/7:TS1 |
| .. |  |
| 127 | N/A in T1 mode |

## E1 Usage Example for Thor-8-PCI-Plus

To run T8pBurstEx in E1 mode with reception of 32 timeslots and transmission on 31 timeslots (TS0 is automatically generated by the transmitter device) the following command line parameters could be used:

T8pBurstEx /me /n2048 /b0           (for board 0)

T8pBurstEx /me /n2048 /b1           (for board 1)

To start receiving data (no file logging) from Li0-Li3

press ‘0’

press ‘r’

To start receiving data (no file logging) from Li4-Li7

press ‘4’

press ‘r’

To start sending a repeated buffer from Li0-Li3

press ‘0’

press ‘s’

To start sending a repeated buffer from Li4-Li7

press ‘4’

press ‘s’

The following buffer is sent from respective Li:

| Li | Send buffer |
| --- | --- |
| 0 | 0x00 – 0x09 |
| 1 | 0x10 – 0x19 |
| 2 | 0x20 – 0x29 |
| 3 | 0x30 – 0x39 |
| 4 | 0x40 – 0x49 |
| 5 | 0x50 – 0x59 |
| 6 | 0x60 – 0x69 |
| 7 | 0x70 – 0x79 |

To view samples of the received data from any of the Li’s

press ‘0’ 		(‘0’ for Li0, ‘1’ for Li1, etc)

press ‘v’ 		(to toggle verbose mode)

The ‘v’ command toggles verbose mode between on and off.

TS0 is the synch timeslot in E1 mode and is handled by the line interface device itself. However, the OTX API allows for overriding the TS0 timeslot contents, if such a feature is required.

Although the received data is not written to a file in this example, the continuous stream of data is read from the OTX driver by consecutive calls to OtxBurstReadData(). The data buffer that is returned by OtxBurstReadData() is formatted modulo 128 bytes in the follow fashion:

| Byte Index | Byte description |
| --- | --- |
| 0 | Li0/4:TS0 (rx) |
| 1 | Li0/4:TS1 |
| 2 | Li0/4:TS2 |
| .. | .. |
| 31 | Li0/4:TS31 |
| 32 | Li1/5:TS0 (rx) |
| 33 | Li1/5:TS1 |
| 34 | Li1/5:TS2 |
| .. | .. |
| 63 | Li1/5:TS31 |
| 64 | Li2/6:TS0 (rx) |
| 65 | Li2/6:TS1 |
| 66 | Li2/6:TS1 |
| .. | .. |
| 96 | Li3/7:TS0 (rx) |
| 97 | Li3/7:TS1 |
| 98 | Li3/7:TS2 |
| .. | .. |
| 127 | Li3/7:TS31 |

## Burst Statistics

There are a number of statistics available to retrieve that can aid in performance profiling, etc.  These are available using the Otx API call OtxDrvGetAttributeValueUint32().  You may reference the Otx SDK documentation for details, as well as the demos T8pBurstEx and OtxBurstStats (a Windows MFC GUI application that can run independently of any running application).