Application Note for Arni6x6Demo

# General

Arni6x6Demo is an example console application demonstrating the use of the phone functionality of the Arni-6x6-PCI-Plus board.

The demo application provides a simple console interface which utilizes simple key-strokes as commands.

The FXS (Foreign Exchange Station) interface is a physical device (OTX_DEVICE_LI_SLIC) and is hosted by the Arni-6x6-PCI-Plus board (OTX_DEVICE_ARNI_6X6_PCI_PLUS).  There are six FXS interfaces on this board. However, this demo does not demonstrate the use of these devices. For a demo of those devices, please see the A66Slic2 demo and the A66FxoFxs demo.

The FXO (Foreign Exchange Office) interface is a logical device (OTX_LDEVICE_LI_POTS) and is hosted by physical DSP device (OTX_DEVICE_DSP). There are six DSP devices on this board, each one of these devices can host one FXO interface.

This demo (in its current shape) configures only one FXO interfaces, and it assumes that the FXO interface is connected to a line simulator, an actual line, or an FXS interface on another board.

The DSP can dynamically host a number of other logical devices (other than the already mentioned OTX_LDEVICE_LI_POTS device) that are used in different phases of a call. These devices are:

- OTX_LDEVICE_CST_DTMF_ DETECTOR

- OTX_LDEVICE_CST_CALLER_ID_DETECTOR

- OTX_LDEVICE_CST_COMFORT_NOISE_GENERATOR

- OTX_LDEVICE_CST_VOICE_ACTIVITY_DETECTOR

- OTX_LDEVICE_CST_LI_POTS

- OTX_LDEVICE_CST_ECHO_CANCEL

- OTX_LDEVICE_CST_DTMF_DIALER

- OTX_LDEVICE_CST_VOICE_MIXER

- OTX_LDEVICE_CST_CPT_GENERATOR

- OTX_LDEVICE_CST_CPT_DETECTOR

The direct sends and receives operate on a highway basis and are hosted by the Burst (OTX_DEVICE_BURST) device.

Pipes are created between the Line interface and the burst device. The pipes act as a logical conduit for the data transfer. All highways on the Arni-6x6-PCI board operates at a 2048kpbs (32 timeslot) speed.

## Pipe Connections when DEMO_USE_DTMF_DIALER defined

The following drawing shows the pipe connections that the application configures when the DEMO_USE_DTMF_DIALER macro is defined:

The following table shows the highway connection for the Arni board:

| Local Highway | Connected To |
| --- | --- |
| 0 | DSP0 |
| 1 | DSP1 |
| 2 | DSP2 |
| 3 | DSP3 |
| 4 | DSP4 |
| 5 | DSP5 |
| 6 | All 6 FXS devices |
| 7 | N/C |
| 8 | Burst |
| 9-15 | N/C |

## Pipe Connections when DEMO_USE_DTMF_DIALER is not defined

The following drawing shows the pipe connections that the application configures when the DEMO_USE_DTMF_DIALER macro is not defined:

# Usage Examples

## Usage Examples

To run Arni6x6Demo execution of the following terminal command line will setup a single Arni-6x6-PCI-Plus board:

Arni6x6Demo

The following command line options are also available:

/L<fileName> 	Log file name (Default = A66FxoFxs.log)

/B<boardNo> 	Board number (Default is 0)

/N<fxoNo> 	FXO number to configure (Default is 2)

/T[A|J]		Board Type used for the demo (Default is A)

A  = Use Arni-6-PCI-Plus Board

J  = Use Arni-6-PCI Board

/D[D|C|V] 	Detector type, where (Default is D)

D  = DTMF Detector

C  = Call Progress Tone Detector

V  = Voice Activity Detector

/C[A|U] 	Voice Compression Codec, where (Default is U)

A  = A-law

U  = u-law

/S[U|G|E|F|A|J]	Country specific setting, where (Default is U)

U  = Li Configuration for USA

G  = Li Configuration for Germany

E  = Li Configuration for United Kingdom

F  = Li Configuration for France

A  = Li Configuration for Australia

J  = Li Configuration for Japan

/O<fileName>	DSP Out file name (for debugging only)

The following command line options are available to allow the user to modify the demo behavior. Most of the option do not require or depend on a sequence of commands. The commands perform their assigned task without having executed a previous command. The data capture using the burst device however does require the commands to use in a given sequence. The sequence needed for the burst data capture is defined later in this section. The command synopsis is shown below and is always available on the demo by entering an ‘h’ while running the demo.

Cmd   Synopsis

======================================================

?    Display Help text

h    Ditto

q    Quit

f    Set active Pots Li Off-Hook

n    Set active Pots Li On-Hook

m    Set active Pots Li in On-Hook Line-Monitor mode

c    Line sense. Read line voltage or loop current

l    Flash/hook on the active Pots Li

r    Read a Register from the DAA

w    Write to a Register in the DAA

b    Get Debug data from the DSP

s    Stream receive data to file (toggle). Only for DSP debugging

e    Enable/Disable Burst (toggle)

a    Abort Burst Senders and Receivers

R    Start Burst Receiver. Prompt for file to recv

!    Terminate Watchdog Timer Thread (Crash test)

v    Toggle Burst Receiver Verbose mode

The following shows the command sequence needed to capture data using the burst device.

Command option ‘e’ to enable the burst device.

>Enable g_hPhysDevs.hBurst

Command option ‘R’ to enter the file name and start the capture.

>BurstReceiver[0]. Please enter file to receive:

Enter the name of the file “testData”.

>BurstReceiver[0]. Please enter single TS[0-31] to receive to file (-1=All) [0]: 0

Enter the time slot, in this demo TS 0 is connected to hPipeRecDaa.

Command option ‘a’ to stop the data capture and close the file..

>DemoBurstReceiver: Closing file