Application Note for A66Slic2

# General

Arni6x6Demo is an example console application demonstrating the use of the phone functionality of the Arni-6x6-PCI-Plus board.

The demo application provides a simple console interface which utilizes single character key-strokes as commands.

The FXS (Foreign Exchange Station) interface is a physical device (OTX_DEVICE_LI_SLIC) and is hosted by the Arni-6x6-PCI-Plus board (OTX_DEVICE_ARNI_6X6_PCI_PLUS).  There are six FXS interfaces on this board. The demo code performs only enough call process and error recovery to show how to connect the FXS interface.

The FXO (Foreign Exchange Office) interface is a logical device (OTX_LDEVICE_LI_POTS) and is hosted by physical DSP device (OTX_DEVICE_DSP). There are six DSP devices on this board, each one of these devices can host one FXO interface. However, this demo does not demonstrate the use of these devices. For a demo of those devices, please see the Arni6x6Demo demo

This demo (in its current shape) configures only one FXS interfaces, and it assumes that the FXS interface is connected to handset, or an FXO interface on another board.

The demo also uses the burst device for data capture on the calling FXS line. This is only used for debug and is not normally enabled. The user can enable the data capture using the console interface.

## Pipe Connections

The following drawing shows the pipe connections that the application configures:

The following table shows the highway connection for the Arni board:

| Local Highway | Connected To |
| --- | --- |
| 0 | DSP0 |
| 1 | DSP1 |
| 2 | DSP2 |
| 3 | DSP3 |
| 4 | DSP4 |
| 5 | DSP5 |
| 6 | All 6 FXS devices |
| 7 | N/C |
| 8 | Burst |
| 9-15 | N/C |

The following table shows the FXS (SLIC) highway 6 time slot connections:

| Time Slot | FXS Interface | Usage |
| --- | --- | --- |
| TS 0 | FXS0 | u-law/a-law/16-bit-linear |
| TS 1 | FXS0 | 16-bit-linear |
| TS 2 | Not used | Not used |
| TS 3 | FXS1 | u-law/a-law/16-bit-linear |
| TS 4 | FXS1 | 16-bit-linear |
| TS 5 | Not used | Not used |
| TS 6 | FXS2 | u-law/a-law/16-bit-linear |
| TS 7 | FXS2 | 16-bit-linear |
| TS 8 | Not used | Not used |
| TS 9 | FXS3 | u-law/a-law/16-bit-linear |
| TS 10 | FXS3 | 16-bit-linear |
| TS 11 | Not used | Not used |
| TS 12 | FXS4 | u-law/a-law/16-bit-linear |
| TS 13 | FXS4 | 16-bit-linear |
| TS 14 | Not used | Not used |
| TS 15 | FXS5 | u-law/a-law/16-bit-linear |
| TS 16 | FXS5 | 16-bit-linear |
| TS 17-31 | Not used | Not used |

# Usage Examples

## Usage Examples

To run A66Slic2 execute the following terminal command line will setup a single Arni-6x6-PCI-Plus board:

A66Slic2

The following command line options are also available:

/L<fileName> 	Log file name (Default = A66Slic2.log)

/B<boardNo> 	Board number (Default is 0)

/N<slicNo> 	Set command line active FXS number 0-5 (Default is 0)

/T[A|J]		Board Type used for the demo (Default is A)

A  = Use Arni-6-PCI-Plus Board

J  = Use Arni-6-PCI Board

The A66Slic demo does not require the user to perform any additional setup. The demo will automatically detect the hook status of the FXS lines and capture the DTMF digits. But the following options are available to allow the user to modify the demo behavior:

?   Display Help text

h   Ditto

q   Quit this demo

0-5 Select active SLIC

a   Set default TS within each Slic

b   Send polarity reversal

c   Set PCM Format

e   Configure Slic Audio Receive Path

M   Enable/Disable Burst (is Disabled)

N   Start Burst Receiver. Prompt for file to recv

O   Abort Burst Receivers

p   Print phone number list

r   Read Slic Direct Register

w   Write Slic Direct Register

R   Read FPGA Register

W   Write FPGA Register

s   Enable CID and ring LI

t   Set each SLIC to a different CID

u   Enter CID strings

U   Ring all %d SLIC Devices with CID

v   Enable Caller ID Send

!   Terminate Watchdog Timer Thread (Crash test)

f   Toggle Fail-over Relay for current Slic

The demo allows the user to enter a phone number for any of the six FXS lines using the handset keypad. The phone numbers available are listed below: Command option p can be used to display the current phone list.

3210 Busy Tone

9876 Reorder Tone

1000 Call FXS Line 0

1001 Call FXS Line 1

1002 Call FXS Line 2

1003 Call FXS Line 3

1004 Call FXS Line 4

1005 Call FXS Line 5

The data capture supported by the burst device and the FXS (SLIC) audio path configuration are the two option most likely needed by the user. The following shows the command sequence needed to capture data using the burst device. The demo connects the calling FXS line to the burst device time slot 0.

Command option ‘M’ to enable the burst device.

>User command > M (0x4d)

>Enable g_hPhysDevs.hBurst

>Open OTX_DEVICE_BURST

>  Connecting FXS 4 to burst device

Command option ‘N’ to enter the file name and start the capture.

>User command > N (0x4e)

>BurstReceiver[0]. Please enter file to receive: BurstReceiver[0].

Enter the name of the file example “testData”.

>Enter single TS[0-31] to receive to file (-1=All):

Enter the burst time slot, in this demo TS 0 is connected to calling FXS line.

Command option ‘O’ to stop the data capture and close the file.

>User command > O (0x4f)

>DemoBurstReceiver: Closing file

The follow show the commands needed to enter the SLIC Audio Path configuration MENU command option e. The user may change these options while the two FXS devices are connected. This allows the user to hear the changes while the line is connected. The SLIC setup menu is displayed every time. Use the command option ‘Q” to exit this menu and return to the main menu.

Command option ‘e’ to start Configure Slic Audio Receive Path.

Cmd Si3210 Audio Receive Path Configuration

The following changes can be made on an active line.

===  ========

Q   Return to Main Menu

0-5 Select active SLIC. #0 is now active

A   Set Transmit Audio Gain

a   Set Receive Audio Gain

b   Set Receive Hybrid Gain

c   Set Receive DC-DC Converter Noise Squelch

d   Set Receive DC-DC Converter Noise Squelch Response

>A

Enter ‘A’ to set transmit audio gain.

>Register value was 0X9

>Gain set to -3.5 dB

>Enter Option

>0: for O db Gain

>1: for -3.5 db Gain

>2: for 3.5 db Gain

>3: Analog receive path muted

>1

Enter ‘1’ to configure the device for a 3.5 dB gain.

>Gain set to -3.5 dB

>Register value is 0X9

>Done