Application Note for T2mRawData

# General

T2mRawData is an example console application demonstrating the use of the DSP to Sending and Receiving data using the OTX Driver with the board's host to DSP interface. This application can be used with the following boards and requires an additional VIDAR DSP daughter card for some board types:

- Thor-8-PCI-Plus (OTX_DEVICE_THOR_8_PCI_PLUS) with VIDAR

- Thor-4-PCI-Plus (OTX_DEVICE_THOR_4_PCI_PLUS) with VIDAR

- Thor-2-PCI-Plus (OTX_DEVICE_THOR_2_PCI_PLUS) with VIDAR

- Gimle-16-PCI-Plus (OTX_DEVICE_GIMLE_16_PCI_PLUS) with VIDAR

- Thor-2-PCMCIA-CST (Thor-2-PCMCIA)

- Thor-2-PCMCIA-EX (Thor-2-PCMCIA)

- Thor-2-PCMCIA-PRO (Thor-2-PCMCIA)

- Thor-2-PCMCIA-Plus (Thor-2-PCMCIA)

- Sleipnir-1-PCI (OTX_DEVICE_SLEIPNIR_1_PCI)

The following VIDAR boards are available to support the raw data transfer:

- Vidar-5x4-ASM-CST

- Vidar-5x4-ASM-EX

- Vidar-5x4-ASM-PRO

- Vidar-55x4-ASM

The following DSP load models are available to support VIDAR boards listed above:

- OtxSpm1

- OtxSpm2

- OtxSpm2Ex

- OtxHdlc

- OtxCst3

The example application provides a console interface which utilizes single character keystroke as the user command interface.

The example code normal configures two T1 or E1 spans, but with Sleipnir only a single span is available. Theses spans can be connected to another T1 or E1 device, or looped back with an external loop back cable.

Please note that this document was current at this time but the example code is modified as needed to support customer needs and may not match the example given within this document.

## T1/E1 Configuration

The T1/E1 line configuration is performed in the DemoInitT1E1ConfigData() function.

In E1 mode the application configures the line interface in CRC4 Multiframe/HBD3 format. In T1 mode the application configures the T1 line interface for ESF/B8ZS frame format. The 100 ohm termination is configured for both T1 and E1.

The configuration can easily be modified to match any possible T1/E1 configuration by editing the settings of the structure members in the DemoInitT1E1ConfigData() function or by use of the command line argument interface. See section “Initialization Command Line Argument” for more details on the command line options available.

## DSP Highway Data Rate

The data rate for each of the DSP highways is determined by the board type and ASM used, in most cases it is set to 2Mbps which supports 32 channels. The Thor-8-PCI-Plus and Thor-4-PCI-Plus provide 2Mbps and 4Mbps option. The Gimle-16-PCI-Plus provides an 8Mbps highway to each DSP which supports 128 channels. The 5x4 Vidar ASM use the 2Mbps highway in all card types except the Gimle-16-PCI-Plus which only supports the 8Mbps highway. The 55x4 Vidar ASM will use the highest data rate available for the given board type.

## Example Application Structure

The entire example application is contained in a single ‘C’ source code file. The file contains a large number of functions used to support the goal of the application but is separated into four part. The following paragraphs will discuss the four parts initialization of global resources, the configuration of physical resources, the configuration of logical resources, and resource control.

### Initialization of Global Resources

The application shares a number of common or global resources within the code.

The main interface with the user is the consol interface. It is initialized using the OtxConsoleInit() command. This single interface is used by all the resources to communicate with the user and allow the user to determine code behavior.

The application provides a single log file. The file name is determined by the define DEMO_DEFAULT_LOG_FILE, but can also be modified using the command line argument /L.

The application uses a single event queue for all physical and logical resources. The event queue is used by the resources to inform the application of changes in resource state or resource activity that may need the application to perform a given task. The event queue is monitored by a separate thread started by a call to OsaThreadStart(). There can be multiple event queue and threads created to ensure that an event is not blocked while another event is being processed. This is especially true for devices that are supplying data and can not wait for a lower priority event to finish processing.

### Configuration of Physical Resources

Physical resources are provided by the hardware. The board types supported by this application each provide a unique set of resources that can not change dynamically. These resources are physical devices with different hardware requirements. The application performs the necessary configuration needed to allow the data transfer and support for the logical devices.

The application provides three functions to perform this task: DemoCreatePhysicalDevices(), DemoConfigureE1T1Devices(), and DemoEnablePhysicalDevices().

The DemoCreatePhysicalDevices() function determines which devices are available with each hardware type and assigns the appropriate handle to allow the device to communicate with the application. It also determines if each device is operating correctly.

The DemoConfigureE1T1Devices() function provides the interface configuration.

The DemoEnablePhysicalDevices() function enables the device once it has been configured and is ready to perform the necessary task.

### Configuration of Logical Resources

The logical devices are differentiated from the physical devices by the fact the number of logical devices is not set. More of them can be requested by the user and allocated by the system if needed. Logical Devices can be classified into Control Device and Data Filter Devices:

- Control Device:  A logical device performing a control function. Like Event Queue.

- Data Filter Device: A device implemented in Hardware, Firmware of Software which converts, processes, or manipulates serial data. Examples of data devices include Time-Space Switch, HDLC Controller, Tone Detector, and Tone Generator.

- Data Filter devices can be categorized into 3 basic types:

  - Data Source: Originates a Pipe or adds data to an existing Pipe. Like DTMF Tone Generator.

  - Data Sink: Terminates a Pipe or extracts data from a Pipe. Like DTMF Detector.

  - Data Bridge: Cross-connects data from one Pipe to another or combines data from multiple Pipes into one Pipe. Like cross-connect in TSS.

The application provides four functions to support the logical device: DemoCreateLogicalDevices(),DemoConnectDevices(),DemoInitializeLogicalDevices(), and DemoEnableLogicalDevices().

The DemoCreateLogicalDevices is similar to the one used to create the physical device. It is used to assign a handle to each device to allow for communication with the device. It is also used to determine if the logical device has the necessary resource available. It should also be noted that multiple logical devices can be supported by the physical device but there is a limit to the number of logical devices a physical device can support.

The DemoConnectDevices performs the connection of the devices using the pipe device. A Pipe Device is a logical entity representing a serial bit-stream and a subset of a Highway with a data rate between 8 kbit/s and 2.048Mbit/s. A pipe is a characterized by its capacity (in 8 kbit/s increments). A pipe is connected to physical or logical devices and mapped to specific time-slots.

The DemoInitializeLogicalDevices is called once the device has been created and connected. It is used to initialize the logical device.

The DemoEnableLogicalDevices function enables the device once it has been configured and is ready to perform the necessary task.

### Resource Control

The application provides the user with a consol interface which allows the user to control the resources. The application also provides an event queue and monitoring thread to control and process data provided by the physical and logical devices. The majority of the processing is performed while in the thread.

## ASM Load Module Options

There are five DSP load models supported by this example application. The table below shows the DSP load module and Vidar ASM options that are supported. In the table the letter ‘S’ is used to indicate the board type with ASM is supported by the load module, if the board type with ASM is not support by the load module the letters ‘NS’ is shown.

| Board type with ASM | OTX SPM1 | OTX SPM2 | OTX SPM2 EX | OTX HDLC | OTX CST3 |
| --- | --- | --- | --- | --- | --- |
| GIMLE_16 with 5x4_CST | NS | NS | NS | NS | NS |
| GIMLE_16 with 5x4_EX | NS | NS | S | S | NS |
| GIMLE_16 with 5x4_PRO | NS | NS | S | S | NS |
| GIMLE_16 with 55x4 | NS | NS | NS | S | NS |
| THOR_8_PCI_plus with 5x4_CST | S | S | NS | S | S |
| THOR_8_PCI_plus with 5x4_EX | S | S | NS | S | NS |
| THOR_8_PCI_plus with 5x4_PRO | S | S | S | S | NS |
| THOR_8_PCI_plus with 55x4 | NS | NS | NS | S | NS |
| THOR_4_PCI_plus with 5x4_CST | S | S | NS | S | S |
| THOR_4_PCI_plus with 5x4_EX | S | S | NS | S | NS |
| THOR_4_PCI_plus with 5x4_PRO | S | S | S | S | NS |
| THOR_4_PCI_plus with 55x4 | NS | NS | NS | S | NS |
| THOR_2_PCI_plus with 5x4_CST | S | S | NS | S | S |
| THOR_2_PCI_plus with 5x4_EX | S | S | NS | S | NS |
| THOR_2_PCI_plus with 5x4_PRO | S | S | S | S | NS |
| THOR_2_PCI_plus with 55x4 | NS | NS | S | S | NS |
| SLEIPNER_1_PCI | NS | NS | S | S | NS |
| Thor-2-PCMCIA-CST | S | S | NS | S | S |
| Thor-2-PCMCIA-EX | S | S | S | S | NS |
| Thor-2-PCMCIA-PRO | S | S | S | S | NS |
| Thor-2-PCMCIA-Plus | S | S | NS | S | NS |

## Initialization Command Line Argument

Command switch  /L<fileName> : Log file name

Command switch  /B<boardNo>  : Board number

(Default is DEMO_DEFAULT_BOARD_NO)

Command switch  /T[2|4|8|M|P|S|T|H|G]: Board Type used for the demo

2 = Use Thor-2-PCI with Vidar-5x4-ASM

4 = Use Thor-4-PCI-Plus with Vidar-5x4 Plus

8 = Use Thor-8-PCI with Vidar-5x4-ASM

M = Use Thor-2-PCMCIA Board

P = Use Thor-8-PCI-Plus with Vidar-5x4 Plus

S = Use Sleipnir-1-PCI (DSP version)

T = Use Thor-2-PCI-Plus with Vidar-5x4 Plus

H = Use Hugin-2-PC104

G = Use Gimle-16-PCI-Plus

Command switch  /C[8|A|U]    : Data format for the RawDataReceiver

8 = No conversion. (Default)

A = A-law converted to 16-bit values in received data stream

U = U-law converted to 16-bit values in received data stream

Command switch  /X[8|A|U]    : Data format for the RawDataSender

8 = No conversion. (Default)

A = Supply 16-bit values, which are converted to A-law by device

U = Supply 16-bit values, which are converted to U-law by device

Command switch  /ME          : E1 Li mode  (Default)

Command switch  /MT          : T1 Li mode

Command switch  /N<capacity> : Raw Data Pipe capacity in kbps

Capacity = 64kbps increments

Command switch  /S<timeslot> : Starting timeslot (default is TS1)

Command switch  /P[H|1|2|X|3] : DSP module

H = OtxHdlc (default)

1 = OtxSpm1

2 = OtxSpm2

X = OtxSpm2Ex

3 = OtxCst3

Command switch  /O[x]        : Clock Source, where

x = 0 through 15 indicating with Li to recover the clock from

x = H to recover from H.100 A-clock (ETCF mode)

Command switch  /A           : Front End Amplifier = ON (default is OFF)

Command switch  /E           : Receive Equalizer = ON (longhaul) (default is OFF)

Command switch  /2           : -20dB MonitorMode = ON (default is OFF)

Command switch  /H           : Receive Input Threshold in mV

(default is DEMO_DEFAULT_RX_THRESHOLD mV)

Command switch  /Y           : Termination Mode

1 = 100/120 ohm balanced (default)

7 = 75 ohm unbalanced

Z = High-impedance

S = 75 ohm balanced

Command switch  /F[E|S|9]    : T1 Frame Format

E = ESF (default)

4 = F4

S = SF [D3/D4] (F12)

9 = SLC96 (F72)

Command switch  /J[A|B]      : T1 Line Code, where

A = AMI

B = B8ZS (default)

Command switch  /G[D|C|G]    : E1 Frame Format

D = Doubleframe

C = CRC4 Multiframe (default)

G = CRC4 Multiframe (G.706)

Command switch  /U[A|H]      : E1 Line Code

A = AMI

H = HDB3 (default)

Command switch  /R              : Raw Data Receiver: octets are formatted lsb received last

Command switch  /Z<str>      : Command string (overrides keyboard input)

Command switch  /Q           : Exit without prompting on errors

# Usage Examples

The following is a list of the commands provided by the example application.

Cmd Synopsis

==========

?   Display Help text

h   Ditto

q   Quit

0-1 Select active Li or Dsp, this Li/Dsp is then used for subsequent commands

x   Send file using RawData Sender[0]. Received by RawData Receiver[0]

a   Abort all Raw Data Sender and Receiver threads

r   Start Raw Data Receiver[0] (no file logging)

R   Start Raw Data Reciever[0]. Prompt for file name

s   Start Raw Data Sender[0]. Continously send a text string

S   Start Raw Data Sender[0]. Prompt for file to send

T   Set test string for continuous data transfer, option s

v   Toggle verbose (print received data) setting Raw Data Receiver[0]

l   Query Line Status of both Lis

t   Toggle verbose buffer usage statistics

g   Enable/Disable Gamma Test

G   Clear Gamma Test error counters

i   Print Gamma Test Results

## E1 Usage Example for Thor-8-PCI-Plus

The following command line will setup the T2mRawData example application for use with a Thor-8-PCI-Plus card (/TP), board number zero (/B0), E1 configuration (/ME). The log file name will be LT8ppRawData.log. The last command sequence “/Z” allows access to the command option provided by the application. Please reference the command synopsis shown in section 2. The first sequence “0r” is used to select line 0, and start receiving data without using a log file. The second sequence “1s” selects line 1 and configures the continuous sending of the text string. The text string is defined by DEMO_TEST_STRING to be “abcdefg” (61, 62, 63, 64, 65, 66, 67). . The ~1000 will place the application into a delay or sleep state for 1000 mSec to allow completion of the current command sequence. The ‘v’ option displays the raw data received.

T2mRawData /TP /B0/ME /LT8ppRawData.log /Z"0r1s1r0sl~1000^0v~500^v1v~500^v"

The log file produced by a Thor-8-PCI-Plus board with a Vidar-5x4-ASM-PRO can be seen below:

Log file is T8PPRAWDATA.LOG

Raw Data Pipe capacity is 64

Library version 2.16.1.0 (filename OtxDrv.Lib, static linked)

Connecting lib...

DemoCreateEventQueue

DemoCreatePhysicalDevices

Opening Thor-8-PCI-Plus board

Open OTX_DEVICE_QUAD_T1E1 #0

Open OTX_DEVICE_QUAD_T1E1 #1

Open OTX_DEVICE_LI_T1E1 #0 (hosted by OTX_DEVICE_QUAD_T1E1 #0)

Open OTX_DEVICE_LI_T1E1 #1 (hosted by OTX_DEVICE_QUAD_T1E1 #0)

Opening Vidar-5x4-ASM-PRO board

DSP#0: Loading DSP program OTX_DSP_SPM_OTXHDLC

DSP#1: Loading DSP program OTX_DSP_SPM_OTXHDLC

Configuring E1/T1 devices...

Configured E1/T1 devices

DemoEnablePhysicalDevices

Enable hThorController0

Enable OTX_DEVICE_QUAD_T1E1 #0 device

Enable OTX_DEVICE_QUAD_T1E1 #1 device

Enable g_hPhysDevs.hLi[0]

Enable g_hPhysDevs.hLi[1]

Enable g_hPhysDevs.hDsp[0]

Enable g_hPhysDevs.hDsp[1]

Clock Source: Line Interface 0

DemoCreateLogicalDevices

RawDataSender[0  22620053]: nIrpQueueLimit = 10

RawDataReceiver[0  22640055]: nFifoSize = 4096 bytes  (sizePerTimeslot=4096)

RawDataSender[1  22660057]: nIrpQueueLimit = 10

RawDataReceiver[1  22680059]: nFifoSize = 4096 bytes  (sizePerTimeslot=4096)

DemoConnectDevices

Creating pipe for RawData Sender/Receiver starting at TS1

Creating pipe for RawData Sender/Receiver starting at TS1

Connecting RawData Sender/Receiver Logical Device #0

Connecting RawData Sender/Receiver Logical Device #1

Finished DemoConnectDevices

DemoInitializeLogicalDevices

DemoEnableLogicalDevices

Enable mRawDataSender[0]

Enable mRawDataSender[1]

06-12 17:07:39.137 E1/T1[0] Event: E1: Physical link ACTIVE indication

06-12 17:07:39.137 E1/T1[0] Event: E1: Loss Of Signal (Red Alarm) Recovery

06-12 17:07:39.137 E1/T1[1] Event: E1: Physical link ACTIVE indication

06-12 17:07:39.137 E1/T1[1] Event: E1: Loss Of Signal (Red Alarm) Recovery

06-12 17:07:39.140 E1/T1[0] Event: E1: MultiFrame Alignment Recovery

06-12 17:07:39.141 E1/T1[1] Event: E1: MultiFrame Alignment Recovery

Press 'h' for help

Key '0' pressed

Li/Dsp#0 Selected ....

Key 'r' pressed

Creating Raw Data Receiver[0] Thread

Key '1' pressed

Li/Dsp#1 Selected ....

Key 's' pressed

DemoRawDataReceiver[0]: Waiting for events. Reading data every 256 milliseconds.

RawDataSender[1]. Continously send text string "abcdefg".

Creating Raw Data Sender[1] Thread

Key '1' pressed

Li/Dsp#1 Selected ....

DemoRawDataSender[1]: Preparing text string buffer...

DemoRawDataSender[1]: Done preparing text string buffer (len=1022)...

DemoRawDataSender[1][22660057]: Primed 10220 bytes

Key 'r' pressed

Creating Raw Data Receiver[1] Thread

Key '0' pressed

Li/Dsp#0 Selected ....

Key 's' pressed

RawDataSender[0]. Continously send text string "abcdefg".

Creating Raw Data Sender[0] Thread

DemoRawDataReceiver[1]: Waiting for events. Reading data every 256 milliseconds.

DemoRawDataSender[0]: Preparing text string buffer...

DemoRawDataSender[0]: Done preparing text string buffer (len=1022)...

DemoRawDataSender[0][22620053]: Primed 10220 bytes

Key 'l' pressed

Line status of Li #0 is OK

Line status of Li #1 is OK

Key '~' pressed

Enter milliseconds to sleep :Sleeping 1000 milliseconds

Key '0' pressed

Li/Dsp#0 Selected ....

Key 'v' pressed

Setting verbose setting to ON for Raw Data Receiver[0] Thread

Key '~' pressed

Enter milliseconds to sleep :Sleeping 500 milliseconds

RawDataReceiver[0] (len=2064) 65 66 67 61 62 63 64 65 66 67 61 62 63 64 65 66

RawDataReceiver[0] (len=2048) 64 65 66 67 61 62 63 64 65 66 67 61 62 63 64 65

Key 'v' pressed

Setting verbose setting to OFF for Raw Data Receiver[0] Thread

Key '1' pressed

Li/Dsp#1 Selected ....

Key 'v' pressed

Setting verbose setting to ON for Raw Data Receiver[1] Thread

Key '~' pressed

Enter milliseconds to sleep :Sleeping 500 milliseconds

RawDataReceiver[1] (len=2048) 61 62 63 64 65 66 67 61 62 63 64 65 66 67 61 62

RawDataReceiver[1] (len=2064) 65 66 67 61 62 63 64 65 66 67 61 62 63 64 65 66

Key 'v' pressed

Setting verbose setting to OFF for Raw Data Receiver[1] Thread

## T1 Usage Example for Thor-8-PCI-Plus

The following command line will setup the T2mRawData example application for use with a Thor-8-PCI-Plus card (/TP), board number zero (/B0), T1 configuration (/MT). The log file name will be LT8ppRawData.log. The last command sequence “/Z” allows access to the command option provided by the application. Please reference the command synopsis shown in section 2. The first sequence “0r” is used to select line 0, and start receiving data without using a log file. The second sequence “1s” selects line 1 and configures the continuous sending of the text string. The ~1000 will place the application into a delay or sleep state for 1000 mSec to allow completion to the current command sequence. The ‘v’ option displays the raw data received.

T2mRawData /TP /B0/MT /LT8ppRawData.log /Z"0r1s1r0sl~1000^0v~500^v1v~500^v"

## GAMMA test

The GAMMA test performs data verification on each byte of the raw data stream. The input to the receiver under test is a count sequence between 0x0-0xFF. The file gamma.raw is available as the data source. The test requires that all data receivers are enabled before starting the test. The menu command ‘g’ is used to start/stop the test and the ‘i’ command is used to display the test results.