Application Note for A66FxoFxs Demo

# General

A66FxoFxs is an example console application demonstrating the use of the phone functionality of the Arni-6x6-PCI-Plus board.

The demo application provides a simple console interface which utilizes single character key-strokes as commands.

The FXS (Foreign Exchange Station) interface is a physical device (OTX_DEVICE_LI_SLIC) and is hosted by the Arni-6x6-PCI-Plus board (OTX_DEVICE_ARNI_6X6_PCI_PLUS). There are six FXS interfaces on this board. This demo performs the necessary setup of the SLICs and device configuration needed to communicate with the FXS interface.

The FXO (Foreign Exchange Office) interface is a logical device (OTX_LDEVICE_LI_POTS) and is hosted by physical DSP device (OTX_DEVICE_DSP). There are six DSP devices on this board, each one of these devices can host one FXO interface. The demo supports only a single FXO connection at a time. It is currently configured to use FXO number 2.

This demo (in its current shape) configures only one FXO interfaces, and it assumes that the FXO interface is connected to a line simulator, an actual line, or an FXS interface on another board. The FXS (2) interface should be connected to a handset, which is used to provide the on/off hook, and DTMF tones used to dial the phone numbers. The FXS interface will detect the off hook state. It will then connect the audio path for the FXS and set the FXO to an of hook state. The handset on the FXS can then be used to interface to the FXO line.

The direct sends and receives operate on a highway basis and are hosted by the Burst (OTX_DEVICE_BURST) device. This demo used the burst device as a debug and diagnostic tool. This is an optional component of the demo and is not needed to perform the communication between the FXO and FXS interface.

Pipes are created between the Line interface and the burst device. The pipes act as a logical conduit for the data transfer. All highways on the Arni-6x6-PCI board operates at a 2048kpbs (32 timeslot) speed.

## Example Application Structure

The entire example application is contained in a single ‘C’ source code file. The file contains a large number of functions used to support the goal of the application but is separated into four part. The following paragraphs will discuss the four parts initialization of global resources, the configuration of physical resources, the configuration of logical resources, and resource control.

### Initialization of Global Resources

The application shares a number of common or global resources within the code.

The main interface with the user is the consol interface. It is initialized using the OtxConsoleInit() command. This single interface is used by all the resources to communicate with the user and allow the user to determine code behavior.

The application provides the use a single log file. The file name is determined by the define DEMO_DEFAULT_LOG_FILE, but can also be modified using the command line argument /L.

The application uses a single event queue for all physical and logical resources. The event queue is used by the resources to inform the application of changes in resource state or resource activity that may need the application to perform a given task. The event queue is monitored by a separate thread started by a call to OsaThreadStart(). There can be multiple event queue and threads created to ensure that an event is not blocked while another event is being processed. This is especially true for devices that are supplying data and can not wait for a lower priority event to finish processing.

### Configuration of Physical Resources

Physical resources are provided by the hardware. The board types supported by this application each provide a unique set of resources that can not change dynamically. These resources are physical devices with different hardware requirements. The application performs the necessary configuration needed to allow the data transfer and support for the logical devices.

The application provides three functions to perform this task: DemoCreatePhysicalDevices(), DemoConfigureE1T1Devices(), and DemoEnablePhysicalDevices().

The DemoCreatePhysicalDevices() function determines which devices are available with each hardware type and assigns the appropriate handle to allow the device to communicate with the application. It also determines if each device is operating correctly.

The DemoConfigureE1T1Devices() function provides the interface configuration.

The DemoEnablePhysicalDevices() function enables the device once it has been configured and is ready to perform the necessary task.

### Configuration of Logical Resources

The logical devices are differentiated from the physical devices by the fact the number of logical devices is not set. More of them can be requested by the user and allocated by the system if needed. Logical Devices can be classified into Control Device and Data Devices:

- Control Device:  A logical device performing a control function. Like Event Queue.

- Data Filter Device: A device implemented in Hardware, Firmware of Software which converts, processes, or manipulates serial data. Examples of data devices include Time-Space Switch, HDLC Controller, Tone Detector, and Tone Generator.

- Data Filter devices can be categorized into 3 basic types:

  - Data Source: Originates a Pipe or adds data to an existing Pipe. Like DTMF Tone Generator.

  - Data Sink: Terminates a Pipe or extracts data from a Pipe. Like DTMF Detector.

  - Data Bridge: Cross-connects data from one Pipe to another or combines data from multiple Pipes into one Pipe. Like cross-connect in TSS.

The application provides four functions to support the logical device: DemoCreateLogicalDevices(),DemoConnectDevices(),DemoInitializeLogicalDevices(), and DemoEnableLogicalDevices().

The DemoCreateLogicalDevices is similar to the one used to create the physical device. It is used to assign a handle to each device to allow for communication with the device. It is also used to determine if the logical device has the necessary resource available. It should also be noted that multiple logical devices can be supported by the physical device but there is a limit to the number of logical devices a physical device can support.

The DemoConnectDevices performs the connection of the devices using the pipe device. A Pipe Device is a logical entity representing a serial bit-stream and a subset of a Highway with a data rate between 8 kbit/s and 2.048Mbit/s. A pipe is a characterized by its capacity (in 8 kbit/s increments). A pipe is connected to physical or logical devices and mapped to specific time-slots.

The DemoInitializeLogicalDevices is called once the device has been created and connected. It is used to initialize the logical device.

The DemoEnableLogicalDevices function enables the device once it has been configured and is ready to perform the necessary task.

### Resource Control

The application provides the user with a consol interface which allows the user to control the resources. The application also provides an event queue and monitoring thread to control and process data provided by the physical and logical devices. The majority of the processing is performed while in the thread.

## Pipe Connections

The following drawing shows the pipe connections that the application configures:

The following table shows the highway connection for the Arni board:

| Local Highway | Connected To |
| --- | --- |
| 0 | DSP0 |
| 1 | DSP1 |
| 2 | DSP2 |
| 3 | DSP3 |
| 4 | DSP4 |
| 5 | DSP5 |
| 6 | All 6 FXS devices |
| 7 | N/C |
| 8 | Burst |
| 9-15 | N/C |

The following table shows the FXS (SLIC) highway 6 time slot connections:

| Time Slot | FXS Interface | Usage |
| --- | --- | --- |
| TS 0 | FXS0 | u-law/a-law/16-bit-linear |
| TS 1 | FXS0 | 16-bit-linear |
| TS 2 | Not used | Not used |
| TS 3 | FXS1 | u-law/a-law/16-bit-linear |
| TS 4 | FXS1 | 16-bit-linear |
| TS 5 | Not used | Not used |
| TS 6 | FXS2 | u-law/a-law/16-bit-linear |
| TS 7 | FXS2 | 16-bit-linear |
| TS 8 | Not used | Not used |
| TS 9 | FXS3 | u-law/a-law/16-bit-linear |
| TS 10 | FXS3 | 16-bit-linear |
| TS 11 | Not used | Not used |
| TS 12 | FXS4 | u-law/a-law/16-bit-linear |
| TS 13 | FXS4 | 16-bit-linear |
| TS 14 | Not used | Not used |
| TS 15 | FXS5 | u-law/a-law/16-bit-linear |
| TS 16 | FXS5 | 16-bit-linear |
| TS 17-31 | Not used | Not used |

# Usage Examples

## Usage Examples

To run A66FxoFxs execute the following terminal command line will setup a single Arni-6x6-PCI-Plus board:

A66FxoFxs

The following command line options are also available:

/L<fileName> 	Log file name (Default = A66FxoFxs.log)

/B<boardNo> 	Board number (Default is 0)

/N<slicNo> 	Set command line active FXS number 0-5 (Default is 0)

/T[A|J]		Board Type used for the demo (Default is A)

A  = Use Arni-6-PCI-Plus Board

J  = Use Arni-6-PCI Board

/C[A|U]		Voice Compression Codec, where (Default is U)

A  = A-law

U  = u-law

/S[U|G|E|F|A|J]	Country specific setting, where (Default is U)

U  = Li Configuration for USA

G  = Li Configuration for Germany

E  = Li Configuration for United Kingdom

F  = Li Configuration for France

A  = Li Configuration for Australia

J  = Li Configuration for Japan

/O<fileName>	DSP Out file name (for debugging only)

The A66FxoFxs demo does not require the user to perform any additional setup. The demo will automatically detect the hook status of the FXS lines, capture the DTMF digits and connect the FXO line. But the following options are available to allow the user to modify the demo behavior:

Cmd   Synopsis

===   ===================================================

?    Display Help text

h    Ditto

q    Quit

f    Set active Pots Li Off-Hook

n    Set active Pots Li On-Hook

m    Set active Pots Li in On-Hook Line-Monitor mode

c    Line sense. Read line voltage or loop current

l    Flash/hook on the active Pots Li

r    Read a Register from the DAA

w    Write to a Register in the DAA

b    Get Debug data from the DSP

s    Stream receive data to file (toggle). Only for DSP debugging

e    Enable/Disable Burst (toggle)

a    Abort Burst Senders and Receivers

R    Start Burst Receiver. Prompt for file to recv

!    Terminate Watchdog Timer Thread (Crash test)

v    Toggle Burst Receiver Verbose mode

z    SLIC Audio path configuration MENU

The data capture supported by the burst device and the FXS (SLIC) audio path configuration are the two option most likely needed by the user. The following shows the commands needed to capture data using the burst device.

Command option ‘e’ to enable the burst device.

>Enable g_hPhysDevs.hBurst

Command option ‘R’ to enter the file name and start the capture.

>BurstReceiver[0]. Please enter file to receive:

Enter the name of the file example “testData”.

>BurstReceiver[0]. Please enter single TS[0-31] to receive to file (-1=All) [0]:

Enter the time slot, in this demo TS 0 is connected to hPotsRx.

Command option ‘a’ to stop the data capture and close the file..

>DemoBurstReceiver: Closing file

The follow show the commands needed to enter the SLIC Audio Path configuration MENU command option z. The SLIC audio configuration is only supported while the FXO and FXS are connected. This allows the user to hear the changes while the line is connected. The SLIC setup menu is displayed every time. Use the command option ‘Q” to exit this menu and return to the main menu.

Command option ‘z’ to start SLIC Audio path configuration MENU.

Cmd Si3210 Audio Receive Path Configuration

The following changes can be made on an active line.

===  ========

Q   Return to Main Menu

0-5 Select active SLIC. #0 is now active

A   Set Transmit Audio Gain

a   Set Receive Audio Gain

b   Set Receive Hybrid Gain

c   Set Receive DC-DC Converter Noise Squelch

d   Set Receive DC-DC Converter Noise Squelch Response

>A

Enter ‘A’ to set transmit audio gain.

Register value was 0X9

Gain set to -3.5 dB

Enter Option

0: for O db Gain

1: for -3.5 db Gain

2: for 3.5 db Gain

3: Analog receive path muted

>1

Enter ‘1’ to configure the device for a 3.5 dB gain.

Gain set to -3.5 dB

Register value is 0X9

Done