Application Note for Arni6x6Demo

# General

Arni6x6Demo is an example console application demonstrating the use of the phone functionality of the Arni-6x6-PCI-Plus board.

The demo application provides a simple console interface which utilizes simple key-strokes as commands.

The FXS (Foreign Exchange Station) interface is a physical device (OTX_DEVICE_LI_SLIC) and is hosted by the Arni-6x6-PCI-Plus board (OTX_DEVICE_ARNI_6X6_PCI_PLUS).  There are six FXS interfaces on this board. However, this demo does not demonstrate the use of these devices. For a demo of those devices, please see the A66Slic2 demo and the A66FxoFxs demo.

The FXO (Foreign Exchange Office) interface is a logical device (OTX_LDEVICE_LI_POTS) and is hosted by physical DSP device (OTX_DEVICE_DSP). There are six DSP devices on this board, each one of these devices can host one FXO interface.

This demo (in its current shape) configures only one FXO interfaces, and it assumes that the FXO interface is connected to a line simulator, an actual line, or an FXS interface on another Arni board.

The DSP can dynamically host a number of other logical devices (other than the already mentioned OTX_LDEVICE_LI_POTS device) that are used in different phases of a call. These devices are:

- OTX_LDEVICE_CST_DTMF_ DETECTOR

- OTX_LDEVICE_CST_CALLER_ID_DETECTOR

- OTX_LDEVICE_CST_COMFORT_NOISE_GENERATOR

- OTX_LDEVICE_CST_VOICE_ACTIVITY_DETECTOR

- OTX_LDEVICE_CST_LI_POTS

- OTX_LDEVICE_CST_ECHO_CANCEL

- OTX_LDEVICE_CST_DTMF_DIALER

- OTX_LDEVICE_CST_VOICE_MIXER

- OTX_LDEVICE_CST_CPT_GENERATOR

- OTX_LDEVICE_CST_CPT_DETECTOR

The direct sends and receives operate on a highway basis and are hosted by the Burst (OTX_DEVICE_BURST) device.

Pipes are created between the Line interface and the burst device. The pipes act as a logical conduit for the data transfer. All highways on the Arni-6x6-PCI board operates at a 2048kpbs (32 timeslot) speed.

The demo defaults to the u-law data compression. The command line option ‘C’ allows the selection of the signed 16 bit linear uncompressed data or the A-law data compression. The 16 bit linear option performs no data compression and provides the user with the full 16 bits of data resolution. This option does require the use of 2 timeslots for each device connection to transmit or receive the 16 bits of data. The u-law and A-law algorithm within the DSP performs a 14 bit uniform 8 kHz PCM data compression which conforms to ITU-T G.726. The DSP always shifts the u-law and A-law compression data 2 bits during data conversion within the DSP to ensure the maximum range within the 16 bit value. This preserves the MSB within the 16 bit data resolution within the DSP. The compressed data only needs a single timeslot per connection to send and receive data, but any file that is played back will be adjusted so that the MSB of the 16 bit word will contain the 14 bits of expanded data.

## Example Application Structure

The entire example application is contained in a single ‘C’ source code file. The file contains a large number of functions used to support the goal of the application but is separated into four part. The following paragraphs will discuss the four parts initialization of global resources, the configuration of physical resources, the configuration of logical resources, and resource control.

### Initialization of Global Resources

The application shares a number of common or global resources within the code.

The main interface with the user is the consol interface. It is initialized using the OtxConsoleInit() command. This single interface is used by all the resources to communicate with the user and allow the user to determine code behavior.

The application provides the use a single log file. The file name is determined by the define DEMO_DEFAULT_LOG_FILE, but can also be modified using the command line argument /L.

The application uses a single event queue for all physical and logical resources. The event queue is used by the resources to inform the application of changes in resource state or resource activity that may need the application to perform a given task. The event queue is monitored by a separate thread started by a call to OsaThreadStart(). There can be multiple event queue and threads created to ensure that an event is not blocked while another event is being processed. This is especially true for devices that are supplying data and can not wait for a lower priority event to finish processing.

### Configuration of Physical Resources

Physical resources are provided by the hardware. The board types supported by this application each provide a unique set of resources that can not change dynamically. These resources are physical devices with different hardware requirements. The application performs the necessary configuration needed to allow the data transfer and support for the logical devices.

The application provides three functions to perform this task: DemoCreatePhysicalDevices(), DemoConfigureE1T1Devices(), and DemoEnablePhysicalDevices().

The DemoCreatePhysicalDevices() function determines which devices are available with each hardware type and assigns the appropriate handle to allow the device to communicate with the application. It also determines if each device is operating correctly.

The DemoConfigureE1T1Devices() function provides the interface configuration.

The DemoEnablePhysicalDevices() function enables the device once it has been configured and is ready to perform the necessary task.

### Configuration of Logical Resources

The logical devices are differentiated from the physical devices by the fact the number of logical devices is not set. More of them can be requested by the user and allocated by the system if needed. Logical Devices can be classified into Control Device and Data Devices:

- Control Device:  A logical device performing a control function. Like Event Queue.

- Data Filter Device: A device implemented in Hardware, Firmware of Software which converts, processes, or manipulates serial data. Examples of data devices include Time-Space Switch, HDLC Controller, Tone Detector, and Tone Generator.

- Data Filter devices can be categorized into 3 basic types:

  - Data Source: Originates a Pipe or adds data to an existing Pipe. Like DTMF Tone Generator.

  - Data Sink: Terminates a Pipe or extracts data from a Pipe. Like DTMF Detector.

  - Data Bridge: Cross-connects data from one Pipe to another or combines data from multiple Pipes into one Pipe. Like cross-connect in TSS.

The application provides four functions to support the logical device: DemoCreateLogicalDevices(),DemoConnectDevices(),DemoInitializeLogicalDevices(), and DemoEnableLogicalDevices().

The DemoCreateLogicalDevices is similar to the one used to create the physical device. It is used to assign a handle to each device to allow for communication with the device. It is also used to determine if the logical device has the necessary resource available. It should also be noted that multiple logical devices can be supported by the physical device but there is a limit to the number of logical devices a physical device can support.

The DemoConnectDevices performs the connection of the devices using the pipe device. A Pipe Device is a logical entity representing a serial bit-stream and a subset of a Highway with a data rate between 8 kbit/s and 2.048Mbit/s. A pipe is a characterized by its capacity (in 8 kbit/s increments). A pipe is connected to physical or logical devices and mapped to specific time-slots.

The DemoInitializeLogicalDevices is called once the device has been created and connected. It is used to initialize the logical device.

The DemoEnableLogicalDevices function enables the device once it has been configured and is ready to perform the necessary task.

### Resource Control

The application provides the user with a consol interface which allows the user to control the resources. The application also provides an event queue and monitoring thread to control and process data provided by the physical and logical devices. The majority of the processing is performed while in the thread.

## Pipe Connections

The following drawing shows the pipe connections that the application configures:

Figure 1 Pipe Connections

Please note the dotted blue lines which denote internal TSS pipe connections. These are useful for creating additional “data sinks” for already existing device pipe connections (solid blue lines). In this example we are sending the following data to the Burst highway:

- TS0-1 - the received samples from the DAA device

- TS2-3 – the data that we are “playing” from the Burst device

- TS4-5 – the output from the DTMF dialer

- TS6-7 – the output from the Voice Mixer

The following table shows the highway connection for the Arni board:

| Local Highway | Connected To |
| --- | --- |
| 0 | DSP0 |
| 1 | DSP1 |
| 2 | DSP2 |
| 3 | DSP3 |
| 4 | DSP4 |
| 5 | DSP5 |
| 6 | All 6 FXS devices |
| 7 | N/C |
| 8 | Burst |
| 9-15 | N/C |

Table 1 Highway Connections

The pipe connections for ARNI are more difficult then any of the other OTX boards. See the explanation of the connections used in the ARNI demo below:

The setup/connection of the ARNI pipes requires the user to know a lot more about how the hardware is connected, the physical connection of the pipe. The hardware has defined that DSP0 will use highway 0, DSP1 will use highway 1…DSP5 will use highway 5. The burst device uses highway 8. These highways all have 32 timeslots.

The other concept that is used on ARNI is “dummy pipe connections”. These are pipe that are used to connect the logical devices within the DSP. The DSP is already connected to the highway but the logical device being supported by the DSP still requires a pipe connection to define the input and/or output. In most cases this is used to assign the timeslot the logical device will use. The DSP already knows which highway it is using. The dummy pipe connection is just used for the internal connection within the DSP and is only used by a single logical device.

The g_hPipes.hMixer1 is being used by the logical device (voice mixer) to define input 1. This pipe is required by the logical device and is only used by this device to define this input.

But if the user wants to make a change to the way the logical devices are connected they would have to close all the pipes as well as the logical devices using the pipes to make any change to the pipe connections. The g_hPipes.hMixer1a and g_hPipes.hMixer1b are used to perform the connection between devices. The TSS device allows the user to create and delete these connections without causing any change to any other device. But to use the TSS device the user needs to know the highway number and timeslot assigned to make the connection.

## System Delays

The diagram below shows the connection as well as the sources of the system delays. The DSP and DAA work on data in packets equal to 10 frames. This gives a minimum of 30 frames of delay for any device within the DSP that has input and output paths. The voice mixer would be a device that would add 32 frames of delay. The 3 data packet being processed and one frame of delay for its input or output. The TSS device always adds 2 frames of delay. The burst device adds a single frame of delay to its input or output. A single frame of data will equal 125mSec of delay.

Figure 2 Data Delay Path

| Device | Delay within device (in frames) |
| --- | --- |
| DAA | 11 |
| PotsLi | 42 |
| TSS | 2 |
| Burst | 1 |
| Voice Mixer | 32 |

Table 2 Frame Delays per Device

The total delay time for the system in the diagram would be 112 frames or 14mSec. The input and output paths are equal in their delays and would take 7mSec. If the voice mixer were used in the output path an additional 34 frames of delay would be added to the path.

# Usage Examples

## Usage Examples

To run Arni6x6Demo execution of the following terminal command line will setup a single Arni-6x6-PCI-Plus board:

Arni6x6Demo

The following command line options are also available:

/L<fileName> 	Log file name (Default = Arni6x6Demo.log)

/B<boardNo> 	Board number (Default is 0)

/N<fxoNo> 	FXO number to configure (Default is 0)

/T[A|J]		Board Type used for the demo (Default is A)

A  = Use Arni-6-PCI-Plus Board

J  = Use Arni-6-PCI Board

/D[D|C|V] 	Detector type, where (Default is D)

D  = DTMF Detector

C  = Call Progress Tone Detector

V  = Voice Activity Detector

/C[A|U] 	Voice Compression Codec, where (Default is U)

A  = A-law

L  = Signed 16 Bit Linear

U  = u-law

/S[U|G|E|F|A|J]	Country specific setting, where (Default is U)

U  = Li Configuration for USA

G  = Li Configuration for Germany

E  = Li Configuration for United Kingdom

F  = Li Configuration for France

A  = Li Configuration for Australia

J  = Li Configuration for Japan

/O<fileName>	DSP Out file name (for debugging only)

The following command line options are available to allow the user to modify the demo behavior. Most of the option do not require or depend on a sequence of commands. The commands perform their assigned task without having executed a previous command. The data capture using the burst device however does require the commands to use in a given sequence. The sequence needed for the burst data capture is defined later in this section. The command synopsis is shown below and is always available on the demo by entering an ‘h’ while running the demo.

Cmd   Synopsis

======================================================

?    Display Help text

h    Ditto

q    Quit

0-5  Select active Line Interface

f    Set active Pots Li Off-Hook

n    Set active Pots Li On-Hook

m    Set active Pots Li in On-Hook Line-Monitor mode

c    Line sense. Read line voltage or loop current

d    Call DTMF digits 201 on the active Pots Li

D    Dial user specified DTMF digits on the active Pots Li

k    Connect DTMF Dialer directly to PotLi (Bypass Voice Mixer)

l    Flash/hook on the active Pots Li

r    Read a Register from the DAA

w    Write to a Register in the DAA

b    Get Debug data from the DSP

s    Stream receive data to file (toggle). Only for DSP debugging

e    Enable/Disable Burst (toggle)

a    Abort Burst Senders and Receivers

R    Start Burst Receiver. Prompt for file to recv

S    Start Burst Sender. Prompt for file to send

L    Send loop. Start sending the file again once EOF is reached.

!    Terminate Watchdog Timer Thread (Crash test)

v    Toggle Burst Receiver Verbose mode

The following shows the command sequence needed to capture data using the burst device.

Command option ‘e’ to enable the burst device.

>Enable g_hPhysDevs.hBurst

Command option ‘R’ to enter the file name and start the capture.

>BurstReceiver[0]. Please enter file to receive:

Enter the name of the file “testData”.

>BurstReceiver[0]. Please enter single TS[0-31] to receive to file (-1=All) [0]: 0

Enter the time slot, in this demo TS 0 is connected to hPipeRecDaa.

Command option ‘a’ to stop the data capture and close the file.

>DemoBurstReceiver: Closing file