Application Note for A66GroundStart

# General

A66GroundStart is an example console application demonstrating the use of the phone functionality of the Arni-6x6-PCI-Plus board. The ground start circuit is an optional circuit provided by the Arni board. The Ground Start Demo also requires the use of the Arni-6x6 Ground Start Harmonica or equivalent hardware needed to provide the external negative 48 volts, voltage source needed for the Ground Start circuit.

The demo application provides a simple console interface which utilizes single character key-stroke for command input.

The FXS (Foreign Exchange Station) interface is a physical device (OTX_DEVICE_LI_SLIC) and is hosted by the Arni-6x6-PCI-Plus board (OTX_DEVICE_ARNI_6X6_PCI_PLUS).  There are six FXS interfaces on this board. This Demo does not demonstrate the use of these devices. The SLIC demo can be used with this demo to demonstrate the use of the Ground Start function, but it would require a second Arni board which is not a requirement for the Ground Start demo. For a demo of those devices, please see the A66Slic2 demo.

The FXO (Foreign Exchange Office) interface is a logical device (OTX_LDEVICE_LI_POTS) and is hosted by physical DSP device (OTX_DEVICE_DSP). There are six DSP devices on this board, each one of these devices can host one FXO interface. However, this demo does not demonstrate the use of these devices. For a demo of those devices, please see the Arni6x6Demo demo

This demo (in its current shape) configures the ground start circuit.

## Example Application Structure

The entire example application is contained in a single ‘C’ source code file. The file contains a large number of functions used to support the goal of the application but is separated into four part. The following paragraphs will discuss the four parts initialization of global resources, the configuration of physical resources, the configuration of logical resources, and resource control.

### Initialization of Global Resources

The application shares a number of common or global resources within the code.

The main interface with the user is the consol interface. It is initialized using the OtxConsoleInit() command. This single interface is used by all the resources to communicate with the user and allow the user to determine code behavior.

The application provides the use a single log file. The file name is determined by the define DEMO_DEFAULT_LOG_FILE, but can also be modified using the command line argument /L.

The application uses a single event queue for all physical and logical resources. The event queue is used by the resources to inform the application of changes in resource state or resource activity that may need the application to perform a given task. The event queue is monitored by a separate thread started by a call to OsaThreadStart(). There can be multiple event queue and threads created to ensure that an event is not blocked while another event is being processed. This is especially true for devices that are supplying data and can not wait for a lower priority event to finish processing.

### Configuration of Physical Resources

Physical resources are provided by the hardware. The board types supported by this application each provide a unique set of resources that can not change dynamically. These resources are physical devices with different hardware requirements. The application performs the necessary configuration needed to allow the data transfer and support for the logical devices.

The application provides three functions to perform this task: DemoCreatePhysicalDevices(), DemoConfigureE1T1Devices(), and DemoEnablePhysicalDevices().

The DemoCreatePhysicalDevices() function determines which devices are available with each hardware type and assigns the appropriate handle to allow the device to communicate with the application. It also determines if each device is operating correctly.

The DemoConfigureE1T1Devices() function provides the interface configuration.

The DemoEnablePhysicalDevices() function enables the device once it has been configured and is ready to perform the necessary task.

### Configuration of Logical Resources

The logical devices are differentiated from the physical devices by the fact the number of logical devices is not set. More of them can be requested by the user and allocated by the system if needed. Logical Devices can be classified into Control Device and Data Devices:

- Control Device:  A logical device performing a control function. Like Event Queue.

- Data Filter Device: A device implemented in Hardware, Firmware of Software which converts, processes, or manipulates serial data. Examples of data devices include Time-Space Switch, HDLC Controller, Tone Detector, and Tone Generator.

- Data Filter devices can be categorized into 3 basic types:

  - Data Source: Originates a Pipe or adds data to an existing Pipe. Like DTMF Tone Generator.

  - Data Sink: Terminates a Pipe or extracts data from a Pipe. Like DTMF Detector.

  - Data Bridge: Cross-connects data from one Pipe to another or combines data from multiple Pipes into one Pipe. Like cross-connect in TSS.

The application provides four functions to support the logical device: DemoCreateLogicalDevices(),DemoConnectDevices(),DemoInitializeLogicalDevices(), and DemoEnableLogicalDevices().

The DemoCreateLogicalDevices is similar to the one used to create the physical device. It is used to assign a handle to each device to allow for communication with the device. It is also used to determine if the logical device has the necessary resource available. It should also be noted that multiple logical devices can be supported by the physical device but there is a limit to the number of logical devices a physical device can support.

The DemoConnectDevices performs the connection of the devices using the pipe device. A Pipe Device is a logical entity representing a serial bit-stream and a subset of a Highway with a data rate between 8 kbit/s and 2.048Mbit/s. A pipe is a characterized by its capacity (in 8 kbit/s increments). A pipe is connected to physical or logical devices and mapped to specific time-slots.

The DemoInitializeLogicalDevices is called once the device has been created and connected. It is used to initialize the logical device.

The DemoEnableLogicalDevices function enables the device once it has been configured and is ready to perform the necessary task.

### Resource Control

The application provides the user with a consol interface which allows the user to control the resources. The application also provides an event queue and monitoring thread to control and process data provided by the physical and logical devices. The majority of the processing is performed while in the thread.

# Usage Examples

The diagram below shows the hardware configuration used in this demo.

Note: on the special Harmonica, black cable connects to GND on Power Supply and white          cable connects to ‘+’.

## Usage Examples

To run A66GroundStart execute the following terminal command line to setup a single Arni-6x6-PCI-Plus board:

A66GroundStart

The following command line options are also available:

/L<fileName> 	Log file name (Default = A66GroundStart.log)

/B<boardNo> 	Board number (Default is 0)

/T[A|J]		Board Type used for the demo (Default is A)

A  = Use Arni-6-PCI-Plus Board

J  = Use Arni-6-PCI Board

G = Use Arni-6-PCI Board with Ground Start

The following menu options are available to allow the user to modify the demo behavior:

?   Display Help text

h   Display Help text

q   Quit this menu

a   Reset GS Circuit

A   Remove reset from GS Circuit

b   Enable GS Circuit

B   Disable GS Circuit

c   Configure GS and Enable all lines

e   Enable RING Ground on a single line

E   Disable RING Ground on a single line

f   Set FXO line for off hook operation

r   Read GS status Register

w   Write GS status Register

x   Read FPGA Register

y   Write FPGA Register

The user will need to configure the GS function using the ‘c’ menu option. This option performs a reset of the GS function, enable the 6 FXO lines for GS, and enable the GS interrupt.

The user will then need to ground (since -48V is needed, ‘+’ in Power Supply is Ground in this case) the TIP signal on one of the FXO lines.

The demo application will perform the event decoding and data processing needed to determine which line was in a Ground Start State. When TIP is grounded, it shows messages which are similar to the following:

04-21 09:28:38.290 Ground Start Event Received 0x3

FPGA interrupt pending 0X3F80C0

GS interrupt pending

GS status was 1 (0X1)

Ground detected on TIP, line 0

The interrupt used to signal the GS is disabled by the driver interrupt service routine. It is required that the application re-enable the GS interrupt once that GS protocol has been completed.

The user can enable the GS RING reply using menu option ‘e’. The user can then remove the ground from the FXO TIP to clear the signal. The menu option ‘E’ will then be used to remove the RING ground.

The Ground Start enable and Ground Start RING enable should be disabled during off hook operation. Menu option ‘f’ can be used to setup the FXO line for off hook.

The user can re-enable the GS function by selecting menu option ‘b’.