Application Note for T8pBurstEx

# General

T8pBurstEx is a simple example console application demonstrating the use of the DMA Direct Sending and Receiving with the OTX Driver using the board's Burst device to access the data using DMA. This application can be used with the following boards:

- Thor-8-PCI-Plus (OTX_DEVICE_THOR_8_PCI_PLUS)

- Thor-4-PCI-Plus (OTX_DEVICE_THOR_4_PCI_PLUS)

- Sleipnir-1-PCI (OTX_DEVICE_SLEIPNIR_1_PCI)

- Thor-2-PCI-Plus

- Thor-2-PCI-Express

- Hugin-2-PC104

- Thor-2-ExpressCard

The demo application provides a simple console interface which utilizes simple keystrokes as commands.

Up to eight T1 or E1 spans are configured. The actual number of configured spans depends on which board the application is running on. The number of spans that each board supports is stored in the g_demoEnv.coLi variable.

The spans can be connected to another T1 or E1 device, or looped with an external loop cable.

The direct sends and receives operate on a highway basis and are hosted by the Burst (OTX_DEVICE_BURST) device.

Pipes are created between the Line interface and the burst device. The pipes act as a logical conduit for the data transfer. The burst device works with highways that have a capacity (speed) that will vary between different types of OTX boards. The burst highway configuration for each of the supported boards is described in separate chapters (see below).

# Burst Highway Timeslot Mapping

This chapter describes the burst highway/timeslot mapping for the boards that are supported by this application.

## Thor-8-PCI-Plus

For Thor-8-PCI-Plus, two 8192 kbps (128 timeslot each) bidirectional highways are available to handle all of the timeslots for each of the eight line interfaces. The following Li <-> Burst device connections are made in transmit and receive direction:

| Li | Burst highway | Burst highway timeslots |
| --- | --- | --- |
| 0 | 0 | TS0 – TS31 |
| 1 | 0 | TS32 – TS63 |
| 2 | 0 | TS64 – TS95 |
| 3 | 0 | TS96 – TS127 |
| 4 | 1 | TS0 – TS31 |
| 5 | 1 | TS32 – TS63 |
| 6 | 1 | TS64 – TS95 |
| 7 | 1 | TS96 – TS127 |

Please note that timeslot-connections on this type of board is completely flexible via the Time-Space device. The table above merely shows the connections that were made in this application program.

## Thor-4-PCI-Plus

For Thor-4-PCI-Plus, one 8192 kbps (128 timeslot) bidirectional highway is available to handle all of the timeslots for each of the four line interfaces. The following Li <-> Burst device connections are made in transmit and receive direction:

| Li | Burst highway | Burst highway timeslots |
| --- | --- | --- |
| 0 | 0 | TS0 – TS31 |
| 1 | 0 | TS32 – TS63 |
| 2 | 0 | TS64 – TS95 |
| 3 | 0 | TS96 – TS127 |

Please note that timeslot-connections on this type of board are completely flexible via the Time-Space device. The table above merely shows the connections that were made in this application program.

## Sleipnir-1-PCI-Plus

For Sleipnir-1-PCI, one 2048 kbps bidirectional highway is available to handle all of the timeslots for the one line interface that is populated on the board. The highway can be directed towards the Li device or towards the DSP device by the settings of three attributes:

- OTX_ATTR_BRD_DSP_SOURCED,

- OTX_ATTR_BRD_FALC_SOURCED, and

- OTX_ATTR_BRD_S2P_SOURCED.

This application connects the burst highway between the Burst device (S2P) and the T1/E1 Li device (Falc) in the following fashion:

| Li | Burst highway | Burst highway timeslots |
| --- | --- | --- |
| 0 | 0 | TS0 – TS31 |

Please note that timeslot-connections on this type of board are fixed. This board is not equipped with a Time-Space switch.

## Thor-2-PCI-Plus and Thor-2-PCI-Express

For the Thor-2-PCI-Plus and Thor-2-PCI-Express boards, two 2048 kbps bidirectional highways are available to handle all of the timeslots for the one line interface that is populated on the board. The following Li <-> Burst device connections are made in transmit and receive direction:

| Li | Burst highway | Burst highway timeslots |
| --- | --- | --- |
| 0 | 0 | TS0 – TS31 |
| 1 | 1 | TS0 – TS31 |

Please note that timeslot-connections on this type of board are completely flexible via the Time-Space device. The table above merely shows the connections that were made in this application program.

## Hugin-2-PC104

For Hugin-2-PC104, two 2048 kbps bidirectional highways are available to handle all of the timeslots for the one line interface that is populated on the board. The highway can be directed towards the Li devices or towards the DSP devices by the settings of three attributes:

- OTX_ATTR_BRD_DSP_SOURCED,

- OTX_ATTR_BRD_FALC_SOURCED, and

- OTX_ATTR_BRD_S2P_SOURCED.

This application connects the burst highway between the Burst devices (S2P) and the T1/E1 Li devices (Falc) in the following fashion:

| Li | Burst highway | Burst highway timeslots |
| --- | --- | --- |
| 0 | 0 | TS0 – TS31 |
| 1 | 1 | TS0 – TS31 |

Please note that timeslot-connections on this type of board are fixed. This board is not equipped with a Time-Space switch.

## Thor-2-ExpressCard

For the Thor-2-ExpressCard, one 8192 kbps bidirectional highways are available to handle all of the timeslots for the one line interface that is populated on the board. The highway can be directed towards the Li devices or towards the DSP devices by the setting of the OTX_ATTR_BRD_FALC_SOURCED attribute. This application connects the burst highway between the Burst devices (S2P) and the T1/E1 Li devices (Falc) in the following fashion:

| Li | Burst highway | Burst highway timeslots |
| --- | --- | --- |
| 0 | 0 | TS0, TS4, TS8, … , TS124 |
| 1 | 0 | TS2, TS6, TS10, … , TS126 |

Please note that timeslot-connections on this type of board are fixed. This board is not equipped with a Time-Space switch. The odd timeslots of the burst highway connect to the DSP device, but are not used in this application.

# Program Structure, Sections, and Threads

The application provides the user with a console interface which allows the user to control the resources. The application also provides an event queue and monitoring thread to control and process data provided by the physical and logical devices. The keyboard commands are performed in the main thread, but the handling of the burst data is handled in a separate thread that wakes up when the driver has an event available.

The entire example application is contained in a single ‘C’ source code file. The file contains a large number of functions used to support the goal of the application but is separated into four part. The following paragraphs will discuss the four parts initialization of global resources, the configuration of physical resources, the configuration of logical resources, and resource control.

## Initialization of Global Resources

The application shares a number of common or global resources within the code.

The main interface with the user is the consol interface. It is initialized using the OtxConsoleInit() command. This single interface is used by all the resources to communicate with the user and allow the user to determine code behavior.

The application provides the use a single log file. The file name is determined by the define DEMO_DEFAULT_LOG_FILE, but can also be modified using the command line argument /L.

The application uses a single event queue for all physical and logical resources. The event queue is used by the resources to inform the application of changes in resource state or resource activity that may need the application to perform a given task. The event queue is monitored by a separate thread started by a call to OsaThreadStart(). There can be multiple event queue and threads created to ensure that an event is not blocked while another event is being processed. This is especially true for devices that are supplying data and can not wait for a lower priority event to finish processing.

## Configuration of Physical Resources

Physical resources are provided by the hardware. The board types supported by this application each provide a unique set of resources that can not change dynamically. These resources are physical devices with different hardware requirements. The application performs the necessary configuration needed to allow the data transfer and support for the logical devices. The application provides three functions to perform this task:

- DemoCreatePhysicalDevices(),

- DemoConfigureE1T1Devices(), and

- DemoEnablePhysicalDevices().

The DemoCreatePhysicalDevices() function determines which devices are available with each hardware type and assigns the appropriate handle to allow the device to communicate with the application. It also determines if each device is operating correctly.

The DemoConfigureE1T1Devices() function provides the interface configuration.

The DemoEnablePhysicalDevices() function enables the device once it has been configured and is ready to perform the necessary task.

## Configuration of Logical Resources

The logical devices are differentiated from the physical devices by the fact the number of logical devices is not set. More of them can be requested by the user and allocated by the system if needed. Logical Devices can be classified into Control Device and Data Devices:

- Control Device:  A logical device performing a control function. Like Event Queue.

- Data Filter Device: A device implemented in Hardware, Firmware of Software which converts, processes, or manipulates serial data. Examples of data devices include Time-Space Switch, HDLC Controller, Tone Detector, and Tone Generator.

Data Filter devices can be categorized into 3 basic types:

- Data Source: Originates a Pipe or adds data to an existing Pipe. Like DTMF Tone Generator.

- Data Sink: Terminates a Pipe or extracts data from a Pipe. Like DTMF Detector.

- Data Bridge: Cross-connects data from one Pipe to another or combines data from multiple Pipes into one Pipe. Like cross-connect in TSS.

The application provides four functions to support the logical device:

- DemoCreateLogicalDevices(),

- DemoConnectDevices(),

- DemoInitializeLogicalDevices(), and

- DemoEnableLogicalDevices().

The DemoCreateLogicalDevices() function is similar to the one used to create the physical device. It is used to assign a handle to each device to allow for communication with the device. It is also used to determine if the logical device has the necessary resource available. It should also be noted that multiple logical devices can be supported by the physical device but there is a limit to the number of logical devices a physical device can support.

The DemoConnectDevices() function performs the connection of the devices using the pipe device. A Pipe Device is a logical entity representing a serial bit-stream and a subset of a Highway with a data rate between 8 kbit/s and 2.048Mbit/s. A pipe is a characterized by its capacity (in 8 kbit/s increments). A pipe is connected to physical or logical devices and mapped to specific time-slots.

The DemoInitializeLogicalDevices() function is called once the device has been created and connected. It is used to initialize the logical device.

The DemoEnableLogicalDevices() function enables the device once it has been configured and is ready to perform the necessary task.

## Handling of Burst data

The Burst data is handled in the event handling thread.

OtxBurstReadData() is called from a thread function called DemoBurstReceiver(). This thread handles 4 Lis (either Li0-Li3, or Li4-Li7). Thus there will be two instances of this thread (one for Li0-Li3 and one for Li4-Li7). The thread is started with the ‘r’ or ‘R’ command.

Starting with OTX Driver Version 2.0, you may now recover a Frame Reference ID and Time from the Burst Device, which can be used for time-stamping frames.  The first call to the Burst device to fill the buffer now is made using the call OtxBurstReadDataEx(), which saves of the frame reference data (which is relevant to the first frame in the returned buffer), then continues to call OtxBurstReadData() to finish filling the local buffer with as much data as is available or can be held.  The frame reference data is then displayed if the verbose setting is active.

OtxBurstSendData() is called from a thread function called DemoBurstSender(). This thread handles 4 Lis (either Li0-Li3, or Li4-Li7). Thus there will be two instances of this thread (one for Li0-Li3 and one for Li4-Li7). The thread is started with the ‘s’ or ‘S’ command.

Starting with OTX Driver Version 2.0, the Burst device can be used in either the existing polling method, or using the new event driven method.  The following outlines the usage of the event driven method (reference the demo T8pBurstEx.c for details).

- Using two new attributes, OTX_ATTR_BURST_RECEIVER_FIFO_TRIGGER and OTX_ATTR_BURST_SENDER_FIFO_TRIGGER, the event signal trigger levels are set to a given amount.  The following example sets the receiver trigger to 16k, thus, enabling an event, OTX_BURST_RECEIVER_EC_DATA_AVAILABLE, to be sent when 16k of data has been received (and will subsequently fire every 16k afterwards).  Conversely, the sender will trigger when 32k of space is available to the application to buffer more data to send with the event OTX_BURST_SENDER_EC_BUF_DEPLETED.

result = OtxDrvSetAttributeValueUint32(hBurst, OTX_ATTR_BURST_RECEIVER_FIFO_TRIGGER,  0x4000);

result = OtxDrvSetAttributeValueUint32(hBurst, OTX_ATTR_BURST_SENDER_FIFO_TRIGGER,    0x8000);

- In lieu of threads that sleep and poll Burst (via OtxBurstReadData[Ex] and OtxBurstSendData[Ex]), the application can wait on the new Burst events using the standard OtxDrvWaitForSingleEvent()API call, and these events will be visible in the event queue pass when the Burst device was opened with a call to the API OtxDrvOpenPhysicalDevice().  When these events are received, the application can process as previously, using calls to OtxBurstReadData() and OtxBurstSendData().  The only change has been how the application handles the data flow, using either a polling or event driven methodology.

## Clock Source

The T8pBurstEx application uses the OTX_CLOCK_SOURCE_INTERNAL clock source, meaning that the board is clock source for the T1 span. The clock source can be changed by modifying the OtxBrdSetClocks() call in the function main(). E.g. if you wish to recover the clock from Li0 change the OtxBrdSetClocks() call to:

OtxBrdSetClocks(

g_hPhysDevs.hThorController0,

OTX_CLOCK_MODE_LOCAL,

OTX_CLOCK_SOURCE_LOCAL_0

);

## T1/E1 Configuration

The T1/E1 line configuration is set in the DemoInitT1E1ConfigData() function.

In E1 mode the application configures the line interface in CRC4 Multiframe/HDB3 frame format.

In T1 mode the application configures the T1 line interface for ESF/B8ZS frame format.

The configuration can easily be modified to match any possible T1/E1 configuration by editing the settings of the struct members in the DemoInitT1E1ConfigData() function.

Many of the elements in this struct can be set by various command line switches.

# Usage Examples

## T1 Usage Example for Thor-8-PCI-Plus

To run T8pBurstEx in T1 mode with reception of all 24 timeslots plus the F-bits and transmission on 24 timeslots (the line interface will automatically generate the outgoing F-bits), the following command line parameters could be used:

T8pBurstEx /mt /n1600 /b0           (for board 0)

T8pBurstEx /mt /n1600 /b1           (for board 1)

To start receiving data (no file logging) from Li0-Li3

press ‘0’

press ‘r’

To start receiving data (no file logging) from Li4-Li7

press ‘4’

press ‘r’

To start sending a repeated buffer from Li0-Li3

press ‘0’

press ‘s’

To start sending a repeated buffer from Li4-Li7

press ‘4’

press ‘s’

Starting with Otx version 2.0, the T8pBurstEx demo has implemented on-the-fly enabling/disabling of the Burst device (using keyboard commands ‘e’ for Enable and ‘d’ for Disable).  With the new methodology of event driven Burst data flow, it is now easier to control the states of the buffers, thus allowing for tighter control of enabling/disabling the Burst device.  Therefore, before the device can actually send and/or receive data, the command ‘e’ must be issued.  The demo will, then, behave accordingly.  A nice feature of this can be the sending/receiving of a file, the application gains tighter control of the data sent/received.

The following buffer is sent from respective Li:

| Li | Send buffer |
| --- | --- |
| 0 | 0x00 – 0x09 |
| 1 | 0x10 – 0x19 |
| 2 | 0x20 – 0x29 |
| 3 | 0x30 – 0x39 |
| 4 | 0x40 – 0x49 |
| 5 | 0x50 – 0x59 |
| 6 | 0x60 – 0x69 |
| 7 | 0x70 – 0x79 |

To view samples of the received data from any of the Li’s

press ‘0’ 		(‘0’ for Li0, ‘1’ for Li1, etc)

press ‘v’ 		(to toggle verbose mode)

The ‘v’ command toggles verbose mode between on and off.

Although the received data is not written to a file in this example, the continuous stream of data is read from the OTX driver by consecutive calls to OtxBurstReadData(). The data buffer that is returned by OtxBurstReadData() is formatted modulo 128 bytes in the follow fashion:

| Byte Index | Byte description |
| --- | --- |
| 0 | Li0/4:FS/DL bit in LSB bit |
| 1 | Li0/4:TS1 |
| 2 | Li0/4:TS2 |
| .. |  |
| 24 | Li0/4:TS24 |
| 25 | N/A in T1 mode |
| .. |  |
| 31 | N/A in T1 mode |
| 32 | Li1/5: FS/DL bit in LSB bit |
| 33 | Li1/5:TS1 |
| 34 | Li1/5:TS2 |
| .. |  |
| 56 | Li1/5:TS24 |
| 57 | N/A in T1 mode |
| .. |  |
| 63 | N/A in T1 mode |
| 64 | Li2/6: FS/DL bit in LSB bit |
| 65 | Li2/6:TS1 |
| .. |  |
| 96 | Li3/7: FS/DL bit in LSB bit |
| 97 | Li3/7:TS1 |
| .. |  |
| 127 | N/A in T1 mode |

## E1 Usage Example for Thor-8-PCI-Plus

To run T8pBurstEx in E1 mode with reception of 32 timeslots and transmission on 31 timeslots (TS0 is automatically generated by the transmitter device) the following command line parameters could be used:

T8pBurstEx /me /n2048 /b0           (for board 0)

T8pBurstEx /me /n2048 /b1           (for board 1)

To start receiving data (no file logging) from Li0-Li3

press ‘0’

press ‘r’

To start receiving data (no file logging) from Li4-Li7

press ‘4’

press ‘r’

To start sending a repeated buffer from Li0-Li3

press ‘0’

press ‘s’

To start sending a repeated buffer from Li4-Li7

press ‘4’

press ‘s’

The following buffer is sent from respective Li:

| Li | Send buffer |
| --- | --- |
| 0 | 0x00 – 0x09 |
| 1 | 0x10 – 0x19 |
| 2 | 0x20 – 0x29 |
| 3 | 0x30 – 0x39 |
| 4 | 0x40 – 0x49 |
| 5 | 0x50 – 0x59 |
| 6 | 0x60 – 0x69 |
| 7 | 0x70 – 0x79 |

To view samples of the received data from any of the Li’s

press ‘0’ 		(‘0’ for Li0, ‘1’ for Li1, etc)

press ‘v’ 		(to toggle verbose mode)

The ‘v’ command toggles verbose mode between on and off.

TS0 is the synch timeslot in E1 mode and is handled by the line interface device itself. However, the OTX API allows for overriding the TS0 timeslot contents, if such a feature is required.

Although the received data is not written to a file in this example, the continuous stream of data is read from the OTX driver by consecutive calls to OtxBurstReadData(). The data buffer that is returned by OtxBurstReadData() is formatted modulo 128 bytes in the follow fashion:

| Byte Index | Byte description |
| --- | --- |
| 0 | Li0/4:TS0 (Rx) |
| 1 | Li0/4:TS1 |
| 2 | Li0/4:TS2 |
| .. | .. |
| 31 | Li0/4:TS31 |
| 32 | Li1/5:TS0 (Rx) |
| 33 | Li1/5:TS1 |
| 34 | Li1/5:TS2 |
| .. | .. |
| 63 | Li1/5:TS31 |
| 64 | Li2/6:TS0 (Rx) |
| 65 | Li2/6:TS1 |
| 66 | Li2/6:TS1 |
| .. | .. |
| 96 | Li3/7:TS0 (Rx) |
| 97 | Li3/7:TS1 |
| 98 | Li3/7:TS2 |
| .. | .. |
| 127 | Li3/7:TS31 |

## Burst Statistics

There are a number of statistics available to retrieve that can aid in performance profiling, etc.  These are available using the OTX OtxDrvGetAttributeValueUint32() function.  You may reference the OTX SDK documentation for details, as well as the demos T8pBurstEx and OtxBurstStats (a Windows MFC GUI application that can run independently of any running application).

OtxBurstStats can be found in the Demos\Windows subdirectory of the OTX SDK. Please see a screenshot below: