Application Note for T2p5x4

# General

T2p5x4 is an example console application demonstrating the use of the DSP to Sending and Receiving data using the OTX Driver and the line interfaces internal HDLC controller. The application also demonstrates the tone detection and generation provided by the DSP. This application can be used with the following boards and requires an additional VIDAR DSP daughter card for some board types:

- Thor-8-PCI-Plus (OTX_DEVICE_THOR_8_PCI_PLUS) with VIDAR

- Thor-4-PCI-Plus (OTX_DEVICE_THOR_4_PCI_PLUS) with VIDAR

- Thor-2-PCI-Plus (OTX_DEVICE_THOR_2_PCI_PLUS) with VIDAR

- Thor-2-PCI-Express (OTX_DEVICE_THOR_2_PCI_EXPRESS) with VIDAR

- Gimle-16-PCI-Plus (OTX_DEVICE_GIMLE_16_PCI_PLUS) with VIDAR

- Thor-2-PCMCIA-Plus (Thor-2-PCMCIA)

- Thor-2-PCMCIA-EX (Thor-2-PCMCIA)

- Thor-2-PCMCIA-PRO (Thor-2-PCMCIA)

- Sleipnir-1-PCI (OTX_DEVICE_SLEIPNIR_1_PCI)

The following VIDAR boards are available to support the raw data transfer:

- Vidar-5x4-ASM-EX

- Vidar-5x4-ASM-PRO

- Vidar-55x4-ASM

The following DSP load models are available to support VIDAR boards listed above:

- OtxSpm1

- OtxSpm2

- OtxSpm2Ex

The example application provides a console interface which utilizes single character keystroke as the user command interface.

The example code normal configures two T1 or E1 spans, but with Sleipnir only a single span is available. Theses spans can be connected to another T1 or E1 device, or looped back with an external loop back cable.

Please note that this document was current at this time but the example code is modified as needed to support customer needs and may not match the example given within this document.

## T1/E1 Configuration

The T1/E1 line configuration is performed in the DemoInitT1E1ConfigData() function.

In E1 mode the application configures the line interface in CRC4 Multiframe/HBD3 format. In T1 mode the application configures the T1 line interface for ESF/B8ZS frame format. The 100/120 ohm termination is configured for both T1 and E1.

The configuration can easily be modified to match any possible T1/E1 configuration by editing the settings of the structure members in the DemoInitT1E1ConfigData() function or by use of the command line argument interface. See section “Initialization Command Line Argument” for more details on the command line options available.

## DSP Highway Data Rate

The data rate for each of the DSP highways is determined by the board type and ASM used, in most cases it is set to 2Mbps which supports 32 channels. The Thor-8-PCI-Plus and Thor-4-PCI-Plus provide 2Mbps and 4Mbps option. The Gimle-16-PCI-Plus provides an 8Mbps highway to each DSP which supports 128 channels. The 5x4 Vidar ASM use the 2Mbps highway in all card types except the Gimle-16-PCI-Plus which only supports the 8Mbps highway. The 55x4 Vidar ASM will use the highest data rate available for the given board type.

## Example Application Structure

The entire example application is contained in a single ‘C’ source code file. The file contains a large number of functions used to support the goal of the application but is separated into four part. The following paragraphs will discuss the four parts initialization of global resources, the configuration of physical resources, the configuration of logical resources, and resource control.

### Initialization of Global Resources

The application shares a number of common or global resources within the code.

The main interface with the user is the consol interface. It is initialized using the OtxConsoleInit() command. This single interface is used by all the resources to communicate with the user and allow the user to determine code behavior.

The application provides a single log file. The file name is determined by the define DEMO_DEFAULT_LOG_FILE, but can also be modified using the command line argument /L.

The application uses a single event queue for all physical and logical resources. The event queue is used by the resources to inform the application of changes in resource state or resource activity that may need the application to perform a given task. The event queue is monitored by a separate thread started by a call to OsaThreadStart(). There can be multiple event queue and threads created to ensure that an event is not blocked while another event is being processed. This is especially true for devices that are supplying data and can not wait for a lower priority event to finish processing.

### Configuration of Physical Resources

Physical resources are provided by the hardware. The board types supported by this application each provide a unique set of resources that can not change dynamically. These resources are physical devices with different hardware requirements. The application performs the necessary configuration needed to allow the data transfer and support for the logical devices.

The application provides three functions to perform this task: DemoCreatePhysicalDevices(), DemoConfigureE1T1Devices(), and DemoEnablePhysicalDevices().

The DemoCreatePhysicalDevices() function determines which devices are available with each hardware type and assigns the appropriate handle to allow the device to communicate with the application. It also determines if each device is operating correctly.

The DemoConfigureE1T1Devices() function provides the interface configuration.

The DemoEnablePhysicalDevices() function enables the device once it has been configured and is ready to perform the necessary task.

### Configuration of Logical Resources

The logical devices are differentiated from the physical devices by the fact the number of logical devices is not set. More of them can be requested by the user and allocated by the system if needed. Logical Devices can be classified into Control Device and Data Filter Devices:

- Control Device:  A logical device performing a control function. Like Event Queue.

- Data Filter Device: A device implemented in Hardware, Firmware of Software which converts, processes, or manipulates serial data. Examples of data devices include Time-Space Switch, HDLC Controller, Tone Detector, and Tone Generator.

  - Data Filter devices can be categorized into 3 basic types:

    - Data Source: Originates a Pipe or adds data to an existing Pipe. Like DTMF Tone Generator.

    - Data Sink: Terminates a Pipe or extracts data from a Pipe. Like DTMF Detector.

    - Data Bridge: Cross-connects data from one Pipe to another or combines data from multiple Pipes into one Pipe. Like cross-connect in TSS.

The application provides four functions to support the logical device: DemoCreateLogicalDevices(),DemoConnectDevices(),DemoInitializeLogicalDevices(), and DemoEnableLogicalDevices().

The DemoCreateLogicalDevices is similar to the one used to create the physical device. It is used to assign a handle to each device to allow for communication with the device. It is also used to determine if the logical device has the necessary resource available. It should also be noted that multiple logical devices can be supported by the physical device but there is a limit to the number of logical devices a physical device can support.

The DemoConnectDevices performs the connection of the devices using the pipe device. A Pipe Device is a logical entity representing a serial bit-stream and a subset of a Highway with a data rate between 8 kbit/s and 2.048Mbit/s. A pipe is a characterized by its capacity (in 8 kbit/s increments). A pipe is connected to physical or logical devices and mapped to specific time-slots.

The DemoInitializeLogicalDevices is called once the device has been created and connected. It is used to initialize the logical device.

The DemoEnableLogicalDevices function enables the device once it has been configured and is ready to perform the necessary task.

### Resource Control

The application provides the user with a consol interface which allows the user to control the resources. The application also provides an event queue and monitoring thread to control and process data provided by the physical and logical devices. The majority of the processing is performed while in the thread.

## ASM Load Module Options

There are three DSP load models supported by this example application. The table below shows the DSP load module and Vidar ASM options that are supported. In the table the letter ‘S’ is used to indicate the board type with ASM is supported by the load module, if the board type with ASM is not support by the load module the letters ‘NS’ is shown.

| Board type with ASM | OTX SPM1 | OTX SPM2 | OTX SPM2EX |
| --- | --- | --- | --- |
| SLEIPNER_1_PCI | NS | S | S |
| GIMLE_16 with 5x4_EX | NS | NS | S |
| GIMLE_16 with 5x4_PRO | NS | NS | S |
| GIMLE_16 with 55x4 | NS | S | NS |
| THOR_8_PCI_plus with 5x4_EX | S | S | NS |
| THOR_8_PCI_plus with 5x4_PRO | S | S | S |
| THOR_8_PCI_plus with 55x4 | NS | S | NS |
| THOR_4_PCI_plus with 5x4_EX | S | S | NS |
| THOR_4_PCI_plus with 5x4_PRO | S | S | S |
| THOR_4_PCI_plus with 55x4 | NS | S | NS |
| THOR_2_PCI_plus with 5x4_EX | S | S | NS |
| THOR_2_PCI_plus with 5x4_PRO | S | S | S |
| THOR_2_PCI_plus with 55x4 | NS | S | S |
| Thor-2-PCMCIA-EX | S | S | S |
| Thor-2-PCMCIA-PRO | S | S | S |
| Thor-2-PCMCIA-Plus | S | S | NS |
| Thor-2-PCMCIA-CST | S | S | NS |

## Initialization Command Line Argument

The following is a list of command line arguments that can be used to configure initialization parameters for the unit under test (UUT). The command line arguments only use upper case characters.

Command switch  /L<fileName> : Log file name

Command switch  /B<boardNo>  : Board number

(Default is DEMO_DEFAULT_BOARD_NO)

Command switch  /T[2|4|8|M|P|S|T|H|G]: Board Type used for the demo

2 = Use Thor-2-PCI with Vidar-5x4-ASM

4 = Use Thor-4-PCI-Plus with Vidar-5x4 Plus

8 = Use Thor-8-PCI with Vidar-5x4-ASM

M = Use Thor-2-PCMCIA Board

P = Use Thor-8-PCI-Plus with Vidar-5x4 Plus

S = Use Sleipnir-1-PCI (DSP version)

T = Use Thor-2-PCI-Plus with Vidar-5x4 Plus

E = Use Thor-2-PCI-Express with Vidar-5x4 Plus

H = Use Hugin-2-PC104

G = Use Gimle-16-PCI-Plus

Command switch  /C[8|A|U] : Data format for the RawDataReceiver

8 = No conversion. (Default)

A = A-law converted to 16-bit values in received data stream

U = U-law converted to 16-bit values in received data stream

Command switch  /ME : E1 Li mode  (Default)

Command switch  /MT : T1 Li mode

Command switch  /N<capacity> : Raw Data Pipe capacity in kbps

Capacity = 64kbps increments

Command switch  /S<timeslot> : Starting timeslot (default is TS1)

Command switch  /P[1|2|X] : DSP module

1 = OtxSpm1

2 = OtxSpm2 (Default in most cases)

X = OtxSpm2Ex

Command switch  /D[1|F|B]  : Use MF Detector

1 = R1 (Default)

F = R2 Forward

B = R2 Backward

Command switch  /A           : Front End Amplifier = ON (default is OFF)

Command switch  /E           : Receive Equalizer = ON (long haul) (default is OFF)

Command switch  /2           : -20dB Monitor Mode = ON (default is OFF)

Command switch  /H           : Receive Input Threshold in mV

(Default is DEMO_DEFAULT_RX_THRESHOLD)

Command switch  /Y           : Termination Mode

1 = 100/120 ohm balanced (default)

7 = 75 ohm unbalanced

Z = High-impedance

S = 75 ohm balanced

Command switch  /X : DSP debug mode. Prompt to start DSP code from Debugger.

Command switch  /Z<str> : Command string (overrides keyboard input)

Command switch  /Q : Exit without prompting on errors

# Usage Examples

The following is a list of the commands provided by the example application.

Cmd  Synopsis

===========

?   Display Help text

h   Ditto

q   Quit

0-1 Select active Li, this line is then used for subsequent commands

t   Send a HDLC frame on the current Li

e   Enable board interrupts

d   Disable board interrupts

a   Sending HDLC frames continuously

g/G Sending a string of DTMF digits on TS1/TS2 (from current Li)

f/F Sending a string of MF digits on TS5/TS6 (from current Li)

i/I Generate a generic tone on Li#1 (TS5/TS6)

j/J Send one DTMF tone on TS5/TS6 (from current Li)

k/K Send one R1 MF tone on TS5/TS6 (from current Li)

r   Disable/Enable DTMF Dialer devices

R   Reset DTMF Dialer device multiple times

>   Increment TS for codecs (current TS=0)

<   Decrement TS for codecs (current TS=0)

## E1 Usage Example for Thor-8-PCI-Plus

The following command line will setup the T2p5x4 example application for use with a Thor-8-PCI-Plus card (/TP), board number zero (/B0), E1 configuration (/ME). The log file name will be T8pp5x4.log. The last command sequence “/Z” allows access to the command option provided by the application. Please reference the command synopsis shown in section 2. The first command sequence “0~1000^” is used to select line 0 and also place the application into a delay or sleep state while the interface hardware settles. The “t” option sends an HDLC frame on the current line which is 0. The frame is also sent on line 2. The “0g1g” command sequence is used to send the DTMF digits on line 0 and line 1.

T2p5x4 /TP /ME /B0 /LT8pp5x4.log /Z"0~1000^0~1000^t1~1000^t~10^t0g1g"

The log file produced by a Thor-8-PCI-Plus board with a Vidar-5x4-ASM-PRO can be seen below. The UUT used a loop back cables on line 0 and 1.

Log file is T8PP5X4.LOG

MF Tone Type isR1

Library version 2.18.1.0 (filename OtxDrv.Lib, static linked)

Connecting lib...

Creating event queue...

Created event queue

Opening physical devices...

Opening Thor-8-PCI-Plus board

Open OTX_DEVICE_TSS

Open OTX_DEVICE_QUAD_T1E1 #0

Open OTX_DEVICE_QUAD_T1E1 #1

Open OTX_DEVICE_LI_T1E1 #0 (hosted by OTX_DEVICE_QUAD_T1E1 #0)

Open OTX_DEVICE_LI_T1E1 #1 (hosted by OTX_DEVICE_QUAD_T1E1 #0)

Opening Vidar-5x4-ASM-PRO board

DSP#0: DSP TMS320C5416

DSP#1: DSP TMS320C5416

DSP#2: DSP TMS320C5416

DSP#3: DSP TMS320C5416

DSP#0: Loading DSP program OTX_DSP_SPM_OTXSPM2

DSP#1: Loading DSP program OTX_DSP_SPM_OTXSPM2

DSP#2: Loading DSP program OTX_DSP_SPM_OTXSPM2

DSP#3: Loading DSP program OTX_DSP_SPM_OTXSPM2

Opened physical devices

Configuring E1/T1 devices...

Configured E1/T1 devices

Enabling physical devices...

Enable OTX_DEVICE_QUAD_T1E1 #0 device

Enable OTX_DEVICE_QUAD_T1E1 #1 device

Init Bsp for Dsp#0

Init Bsp for Dsp#1

Init Bsp for Dsp#2

Init Bsp for Dsp#3

Creating logical devices...

Created logical devices

MF Detector[0]: OTX_ATTR_MF_DETECTOR_THRESHOLD_TONE_ENERGY  = 1250

MF Detector[0]: OTX_ATTR_MF_DETECTOR_THRESHOLD_PAUSE_ENERGY = 200

MF Detector[0]: OTX_ATTR_MF_DETECTOR_THRESHOLD_TWIST        = 120

MF Detector[0]: OTX_ATTR_MF_DETECTOR_THRESHOLD_RELATIVE_3RD = 400

01C68F0401B3125A E1/T1[0] Event: E1: MultiFrame Alignment Recovery

01C68F0401B33777 E1/T1[1] Event: E1: MultiFrame Alignment Recovery

01C68F0401B3CCB0 E1/T1[0] Event: E1: Alarm Indication Signal (Blue Alarm) Recovery

01C68F0401B3CE09 E1/T1[1] Event: E1: Alarm Indication Signal (Blue Alarm) Recovery

01C68F0401B3E2BC E1/T1[0] Event: E1: Frame Alignment Recovery

01C68F0401B3E309 E1/T1[1] Event: E1: Frame Alignment Recovery

01C68F0401B44966 E1/T1[0] Event: E1: Alarm Indication Signal (Blue Alarm) Recovery

01C68F0401B44BF2 E1/T1[1] Event: E1: Alarm Indication Signal (Blue Alarm) Recovery

01C68F0401B458D2 E1/T1[0] Event: E1: Frame Alignment Recovery

01C68F0401B462A3 E1/T1[1] Event: E1: Frame Alignment Recovery

01C68F0401B4E52D E1/T1[0] Event: E1: MultiFrame Alignment Recovery

01C68F0401B50C3E E1/T1[1] Event: E1: MultiFrame Alignment Recovery

Press 'h' for help

LI#0 Selected ....

Enter milliseconds to sleep :Sleeping 1000 milliseconds

01C68F04024CDFBF E1/T1[0] Event: E1: Receive Slip Negative (2)

01C68F04024CDFD9 E1/T1[0] Event: E1: Errored second

01C68F04024CE00C E1/T1[0] Event: E1: Physical link ACTIVE indication

01C68F04024CE013 E1/T1[0] Event: E1: Loss Of Signal (Red Alarm) Recovery

01C68F04024CE199 E1/T1[1] Event: E1: Code Violation Count (2)

01C68F04024CE243 E1/T1[1] Event: E1: Receive Slip Positive (1)

01C68F04024CE24B E1/T1[1] Event: E1: Errored second

01C68F04024CE279 E1/T1[1] Event: E1: Physical link ACTIVE indication

01C68F04024CE280 E1/T1[1] Event: E1: Loss Of Signal (Red Alarm) Recovery

LI#0 Selected ....

Enter milliseconds to sleep :Sleeping 1000 milliseconds

Sending message on Li #0 ...Done.

LI#1 Selected ....

Enter milliseconds to sleep :Sleeping 1000 milliseconds

01C68F040314052D HDLC Frame Received, Sequence Number = 1

HdlcReceiver[0] Len=10, Fe=0x00, Sq=1: 00 01 02 03 04 05 06 07 08 09

Sending message on Li #1 ...Done.

Enter milliseconds to sleep :Sleeping 10 milliseconds

01C68F0403ACC1CB HDLC Frame Received, Sequence Number = 1

HdlcReceiver[1] Len=10, Fe=0x00, Sq=1: 00 01 02 03 04 05 06 07 08 09

Sending message on Li #1 ...Done.

LI#0 Selected ....

5 Sending digits on Li#0 TS1 '0123456789ABCD*#'

Done.

LI#1 Selected ....

5 Sending digits on Li#1 TS1 '0123456789ABCD*#'

01C68F0403AE614E Done.

HDLC Frame Received, Sequence Number = 2

HdlcReceiver[1] Len=10, Fe=0x00, Sq=2: 00 01 02 03 04 05 06 07 08 09

01C68F0403BB5E0B DtmfDetector[0] detected DTMF digit 0

01C68F0403BB87F0 DtmfDetector[1] detected DTMF digit 0

01C68F0403DC70BD DtmfDetector[0] detected DTMF digit 1

01C68F0403DC9A8C DtmfDetector[1] detected DTMF digit 1

01C68F0403FD8351 DtmfDetector[0] detected DTMF digit 2

01C68F0403FDAD21 DtmfDetector[1] detected DTMF digit 2

01C68F04041E95CC DtmfDetector[0] detected DTMF digit 3

01C68F04041EBFAE DtmfDetector[1] detected DTMF digit 3

01C68F04043FA860 DtmfDetector[0] detected DTMF digit 4

01C68F04043FD23F DtmfDetector[1] detected DTMF digit 4

01C68F040460BB04 DtmfDetector[0] detected DTMF digit 5

01C68F040460E4DC DtmfDetector[1] detected DTMF digit 5

01C68F040481CD87 DtmfDetector[0] detected DTMF digit 6

01C68F040481F76C DtmfDetector[1] detected DTMF digit 6

01C68F0404A2E02D DtmfDetector[0] detected DTMF digit 7

01C68F0404A30A09 DtmfDetector[1] detected DTMF digit 7

01C68F0404C3F2BE DtmfDetector[0] detected DTMF digit 8

01C68F0404C41C9F DtmfDetector[1] detected DTMF digit 8

01C68F0404E50549 DtmfDetector[0] detected DTMF digit 9

01C68F0404E52F3C DtmfDetector[1] detected DTMF digit 9

01C68F04050617E0 DtmfDetector[0] detected DTMF key A

01C68F04050641C8 DtmfDetector[1] detected DTMF key A

01C68F0405272A97 DtmfDetector[0] detected DTMF key B

01C68F0405275467 DtmfDetector[1] detected DTMF key B

01C68F0405483CEC DtmfDetector[0] detected DTMF key C

01C68F04054866FA DtmfDetector[1] detected DTMF key C

01C68F0405694FBE DtmfDetector[0] detected DTMF key D

01C68F04056979A6 DtmfDetector[1] detected DTMF key D

01C68F04058A6231 DtmfDetector[0] detected DTMF key '*'

01C68F04058A8C99 DtmfDetector[1] detected DTMF key '*'

01C68F0405AB750B DtmfDetector[0] detected DTMF key '#'

01C68F0405AB9EBA DtmfDetector[1] detected DTMF key '#'

01C68F0405BEE471 DtmfDialler[0] Event: DTMF Dialling completed

01C68F0405BF0E4E DtmfDialler[1] Event: DTMF Dialling completed

## T1 Usage Example for Thor-8-PCI-Plus

The following command line will setup the T2p5x4 example application for use with a Thor-8-PCI-Plus card (/TP), board number zero (/B0), T1 configuration (/MT). The log file name will be T8pp5x4.log. The last command sequence “/Z” allows access to the command option provided by the application. Please reference the command synopsis shown in section 2. The first command sequence “0~1000^” is used to select line 0 and also place the application into a delay or sleep state while the interface hardware settles. The “t” option sends an HDLC frame on the current line which is 0. The frame is also sent on line 2. The “0g1g” command sequence is used to send the DTMF digits on line 0 and line 1.

T2p5x4 /TP /MT /B0 /LT8pp5x4.log /Z"0~1000^0~1000^t1~1000^t~10^t0g1g"