Application Note for T2p5x4Hdlc

# General

T2p5x4Hdlc is an example console application demonstrating the use of the DSP HDLC logical device used to Sending and Receiving. This application can be used with the following boards and requires an additional VIDAR DSP daughter card for some board types:

- Thor-8-PCI-Plus (OTX_DEVICE_THOR_8_PCI_PLUS) with VIDAR

- Thor-4-PCI-Plus (OTX_DEVICE_THOR_4_PCI_PLUS) with VIDAR

- Thor-2-PCI-Plus (OTX_DEVICE_THOR_2_PCI_PLUS) with VIDAR

- Thor-2-PCI-Express (OTX_DEVICE_THOR_2_PCI_EXPRESS) with VIDAR

- Gimle-16-PCI-Plus (OTX_DEVICE_GIMLE_16_PCI_PLUS) with VIDAR

- Thor-2-PCMCIA-EX (Thor-2-PCMCIA)

- Thor-2-PCMCIA-PRO (Thor-2-PCMCIA)

- Sleipnir-1-PCI (OTX_DEVICE_SLEIPNIR_1_PCI)

The following VIDAR boards are available to support the raw data transfer:

- Vidar-5x4-ASM-EX (TI-5410 DSP)

- Vidar-5x4-ASM-PRO (TI-5416 DSP)

- Vidar-5x4-ASM-CST (TI-54CST DSP)

- Vidar-55x4-ASM (TI-5510 DSP)

The following DSP load models are available to support VIDAR boards listed above:

- OtxHdlc

- OtxSpm1

- OtxSpm2

- OtxSpm2Ex

- OtxHdlcEx

- OtxCst3

The example code normal configures two T1 or E1 HDLC devices each assigned to a different DSP and span, but with Sleipnir only a single DSP and span is available. Theses spans can be connected to another T1 or E1 device, or looped back with an external loop back cable.

The example application provides a console interface which utilizes single character keystroke as the user command interface.

Please note that this document was current at this time but the example code is modified as needed to support customer needs and may not match the example given within this document.

## Application Code Organization

The entire T2p5x4Hdlc example application is contained in a single ‘C’-language source file. The file contains a large number of functions used to demonstrate various features but the code is mainly separated into four parts. The following paragraphs will discuss these four parts in further detail: initialization of global resources, the configuration of physical resources, the configuration of logical resources, and resource control.

### Initialization of Global Resources

The application shares a number of common or global resources within the code.

The main interface with the user is the console interface. It is initialized using the OtxConsoleInit() command. This single interface is used by all the resources to communicate with the user and allow the user to determine code behavior.

The application provides the use a single log file. The file name is determined by the define DEMO_DEFAULT_LOG_FILE, but can also be modified using the command line argument /L.

Another important command line switch is the /T switch. This switch provides the type of OTX target board that the application should use.

However, although the application is written to support this particular set of OTX boards, the application can easily be modified to support any OTX T1/E1 board with DSP resources.

The application uses a single event queue for all physical and logical resources. The event queue is used by the resources to inform the application of changes in resource state or resource activity that may need the application to perform a given task. The event queue is monitored by a separate thread started by a call to OsaThreadStart(). There can be multiple event queue and threads created to ensure that an event is not blocked while another event is being processed. This is especially true for devices that are supplying data and can not wait for a lower priority event to finish processing.

### Configuration of Physical Resources

Physical resources are provided by the hardware. The board types supported by this application each provide a unique set of resources that can not change dynamically. These resources are physical devices with different hardware requirements. The application performs the necessary configuration needed to allow the data transfer and support for the logical devices.

The application provides three functions to perform this task: DemoCreatePhysicalDevices(), DemoConfigureE1T1Devices(), and DemoEnablePhysicalDevices().

The DemoCreatePhysicalDevices() function determines which devices are available with each hardware type and assigns the appropriate handle to allow the device to communicate with the application. It also determines if each device is operating correctly.

The DemoConfigureE1T1Devices() function provides the interface configuration.

The DemoEnablePhysicalDevices() function enables the device once it has been configured and is ready to perform the necessary task.

Two T1 or E1 spans are configured. The T1/E1 Line Interface devices are referred to as Line Interface (Li) devices. All of the OTX device numbers use a zero-count terminology. The two Li devices are therefore called Li0 and Li1.

The configuration parameters for the two spans are set to hard-coded values in the DemoInitT1E1ConfigData() function. The spans can be connected to another T1 or E1 device, or looped with an external loop cable.

This application programs loads the OTX_DSP_SPM_OTXHDLC program module into the DSPs.

### Configuration of Logical Resources

The logical devices are differentiated from the physical devices by the fact the number of logical devices is not set. More of them can be requested by the user and allocated by the system if needed. Logical Devices can be classified into Control Device and Data Devices:

- Control Device:  A logical device performing a control function. Like Event Queue.

- Data Filter Device: A device implemented in Hardware, Firmware of Software which converts, processes, or manipulates serial data. Examples of data devices include Time-Space Switch, HDLC Controller, Tone Detector, and Tone Generator.

- Data Filter devices can be categorized into 3 basic types:

  - Data Source: Originates a Pipe or adds data to an existing Pipe. Like a Sinewave Tone Generator.

  - Data Sink: Terminates a Pipe or extracts data from a Pipe. Like an MF Detector.

  - Data Bridge: Cross-connects data from one Pipe to another or combines data from multiple Pipes into one Pipe. Like cross-connect in TSS.

The application provides four functions to support the logical device: DemoCreateLogicalDevices(), DemoConnectDevices(), DemoInitializeLogicalDevices(), and DemoEnableLogicalDevices().

The DemoCreateLogicalDevices is similar to the one used to create the physical device. It is used to assign a handle to each device to allow for communication with the device. It is also used to determine if the logical device has the necessary resource available. It should also be noted that multiple logical devices can be supported by the physical device but there is a limit to the number of logical devices a physical device can support.

The DemoConnectDevices performs the connection of the devices using the pipe device. A Pipe Device is a logical entity representing a serial bit-stream and a subset of a Highway with a data rate between 8 kbit/s and 2.048Mbit/s. A pipe is a characterized by its capacity (in 8 kbit/s increments). A pipe is connected to physical or logical devices and mapped to specific time-slots. This application uses pipes with a data rate of 64 kpbs, which corresponds to one timeslot of the T1/E1 span.

The DemoInitializeLogicalDevices is called once the device has been created and connected. It is used to initialize the logical device.

The DemoEnableLogicalDevices function enables the device once it has been configured and is ready to perform the necessary task.

### Resource Control

The application provides the user with a consol interface which allows the user to control the resources. The application also provides an event queue and monitoring thread to control and process data provided by the physical and logical devices. The majority of the processing is performed while in the thread.

## Clock Source

The T2p5x4Hdlc application uses the OTX_CLOCK_SOURCE_LOCAL_0 clock source, meaning that the board will synchronize its clock rate to the data rate of the incoming E1 signal of Li0. The clock source can be changed by modifying the OtxBrdSetClocks() call in the function main(). E.g if to become a clock master for the E1 spans, change the OtxBrdSetClocks() call to:

OtxBrdSetClocks(

g_hPhysDevs.hThorController0,

OTX_CLOCK_MODE_LOCAL,

OTX_CLOCK_SOURCE_INTERNAL

);

## T1/E1 Configuration

The T1/E1 line configuration is set in the DemoInitT1E1ConfigData() function.

In E1 mode the application configures the line interface in CRC4 Multiframe/HBD3 format.

In T1 mode the application configures the T1 line interface for ESF/B8ZS frame format.

The configuration can easily be modified to match any possible T1/E1 configuration by editing the settings of the structure members in the DemoInitT1E1ConfigData() function.

## ASM Load Module Options

There are six DSP load models supported by this example application. The table below shows the DSP load module and Vidar ASM options that are supported. In the table the letter ‘S’ is used to indicate the board type with ASM is supported by the load module, if the board type with ASM is not support by the load module the letters ‘NS’ is shown.

| Board type with ASM | OTX SPM1 | OTX SPM2 | OTX HDLC | OTX CST3 | OTX SPM2EX | OTX HDLCEX |
| --- | --- | --- | --- | --- | --- | --- |
| SLEIPNER_1_PCI | NS | S | S | NS | S | S |
| THOR_8_PCI_plus with 5x4_EX | S | S | S | NS | S | NS |
| THOR_8_PCI_plus with 5x4_PRO | S | S | S | NS | S | S |
| THOR_8_PCI_plus with 55x4 | NS | S | S | NS | NS | S |
| THOR_4_PCI_plus with 5x4_EX | S | S | S | NS | S | NS |
| THOR_4_PCI_plus with 5x4_PRO | S | S | S | NS | S | S |
| THOR_4_PCI_plus with 55x4 | NS | S | S | NS | NS | S |
| THOR_2_PCI_plus with 5x4_EX | S | S | S | NS | S | NS |
| THOR_2_PCI_plus with 5x4_PRO | S | S | S | NS | S | S |
| THOR_2_PCI_plus with 55x4 | NS | S | S | NS | NS | S |
| Thor-2-PCMCIA-EX | S | S | S | NS | S | S |
| Thor-2-PCMCIA-PRO | S | S | S | NS | S | S |
| Thor-2-PCMCIA-CST | S | S | S | S | NS | NS |

## System Delays

In most application the delays introduced by the individual devices that live on the board is not a concern. But if an application needs to determine the amount of delay each device will introduce into the system the following diagram and table can be used to calculate the system delay. The diagram below shows the data path as well as the sources of the system delays for the HDLC DSP transmit data packet using a Thor-2-PCI-Plus card. The TSS device always adds 2 frames of delay. The FALC will add 1 frame of delay. A single frame of data will equal 125microSec of delay.

Figure 1 Data Delay Path

| Device | Delay within device (in frames) |
| --- | --- |
| FALC | 1 |
| TSS | 2 |
| Burst | 1 |

Table 1 Frame Delays per Device

The HDLC data frame delay times for though the system in the diagram would be 3 frames or 375microSec.

## Initialization Command Line Argument

The following is a list of command line arguments that can be used to configure initialization parameters for the unit under test (UUT). The command line arguments only use upper case characters.

Command line switches:

/B<boardNo>   : Board number (Default is 0)

/T[2|T|E|M|4|8|P|G|S]  : Board Type used for the demo, where

2 = Use Thor-2-PCI with Vidar-ASM

T = Use Thor-2-PCI-Plus with Vidar-ASM Board

E = Use Thor-2-PCI-Express with Vidar-ASM Board

M = Use Thor-2-PCMCIA

4 = Use Thor-4-PCI-Plus with Vidar-ASM Board

8 = Use Thor-8-PCI with Vidar-ASM Board

P = Use Thor-8-PCI-Plus with Vidar-ASM Board

G = Use Gimle-16-PCI-Plus with Vidar-ASM Board

S = Use Sleipnir-1-PCI (DSP version)

H = Use Hugin-2-PC104

/L<fileName>  : Log file name

/S  : SS7 FISU filtering active

/ME  : E1 Li mode

/MT  : T1/J1 Li mode

/D  : DSP Debugger mode. Prompt to start DSP code from Debugger

/I  : Use HDLC Bit Inversion (Default is to not invert the bits)

/5  : Use HDLC 56kbps channel.

/X  : Use HDLC Super-channels (higher than 64kbps capacity). Only for OTXHDLC.

/N<x>  : Number of 64kbps timeslots in the Super-channels (only used if /X is used)

/V<ts>  : E1/T1 Timeslot to connect the HDLC device to (for 64 or 56 kbps capacity)

/P[1|2|X|C|H|E] : DSP program to load, where

1 = OtxSpm1

2 = OtxSpm2

X = OtxSpm2Ex

H = OtxHdlc (default)

E = OtxHdlcEx (for 5416 and 5510 DSPs)

3 = OtxCst3 (for 54CST DSPs)

/F[E|4|S|9|J|5|F|0] : T1/J1 Frame Format, where

E = ESF (default)

4 = F4

S = SF [D3/D4] (F12)

9 = SLC96 (F72)

J = J1_ESF

5 = J1_F4

F = J1_SF

0 = J1_SLC96

/J[A|B]  : T1/J1 Line Code, where

A = AMI

B = B8ZS (default)

/G[D|C|G]  : E1 Frame Format, where

D = Doubleframe

C = CRC4 Multiframe (default)

G = CRC4 Multiframe (G.706)

/U[A|H]  : E1 Line Code, where

A = AMI

H = HDB3 (default)

/Z<str>  : Command string (overrides keyboard input)

/Q  : Exit without prompting on errors

/K[x]  : Clock source, where

x = I Internal clock source (default) or

x = 0 through 7 indicating with Li to recover the clock from

x = H to recover from H.100 A-clock (ETCF mode)

# Usage Examples

The following test examples requires that Li0 is looped to Li1, using a loop back cable.

## E1 Usage Example for Thor-2-PCI-Plus

To run T2p5x4Hdlc in E1 mode using the Thor-2-PCI-Plus board, the following command line parameters could be used:

T2p5x4Hdlc /me /tt /b0           (for board 0)

T2p5x4Hdlc /me /tt /b1           (for board 1)

To Press ‘h’ to get a list of all possible key commands.

Key h pressed

Cmd  Synopsis (Current HDLC device 0)

===  ========

?   Display Help text

h   Ditto

q   Quit

a   Setting frame sent attribute for current HDLC Sender

t   Send an HDLC frame from current HDLC Sender

f   Start sending FISUs (continuosly) from current HDLC Sender

i   Start sending LSSUs (continuosly) from current HDLC Sender

m   Send an MSU from current HDLC Sender

F   Toggle on/off OTX_HDLC_SS7_FILTER_FISU_ALL filter current HDLC Sender

+   Set number of consecutive frame to be send from the HDLC Sender

S   Set time to sleep (in ms) between transmitted HDLC Frames (0)

L   Set HDLC Frame length to be transmitted

c   Set Clock Source

l   Query Line Status of the two Lis

s   Display the SS7 HDLC receive status of the active HDLC Receiver

n   Increment current HDLC Device (currently HdlcDev#0 selected

N   Decrement current HDLC Device (currently HdlcDev#0 selected

y   Query HDLC Receiver Fifo diagnostics (for current HDLC device)

p   Reset current HDLC Receiver

v   Toggle verbose logging of received HDLC frames

V   Print counters of received HDLC frames

w   Toggle payload checking of received frames

The following command is used to display the line status. If the lines are correctly connected the status returned by the command will be OK.

press ‘l’

## HDLC Send Data Timestamp Example for Thor-2-PCI-Plus

The following command sequence demonstrates how the HDLC send data timestamp can be used. The timestamp is not normally enabled by the HDLC device within the DSP. It is used to determine when the HDLC device within the DSP has finished processing an HDLC packet. There are two events associated with the HDLC send timestamp. The first is the OTX_HDLC_SENDER_EC_FRAME_COUNT_ZERO. It is used to indicate the 16 bit frame count has rolled over and the count is again zero. It is used to synchronize the user application and to allow the application to keep track of the number of frames sent. The 16 bit frame count will be incremented every 125microSec and will rollover every 8.192 seconds. The OTX_HDLC_SENDER_EC_FRAME_SENT event  is the second event associated with the timestamp. This event is generated by the DSP HDLC device when an HDLC frame is completely processed. The frame count is contained within the lower 16 bit of the pEventData->m_nParam data. The m_nParam value is a 32 bit variable passed with the event. The example below requires the user to loop back Li 0 transmit and receive using a loop back cable. Data is transmitted on Li0 and received on Li1.

- The user will need to select the HDLC device. The demo application configures two HDLC devices. The demo application defaults to device 0. Device 1 can be selected by using command option ‘n’. Note this example used the default selection.

- The user will then need to enable the timestamp event within the HDLC device. This is performed by selecting the command option ‘a’. The demo application will initialize the g_coHdlcFramesSentRollOverCount global variable to a negative one. The first rollover event generated by the DSP will then set the value to zero. The following text should be displayed:

Key a pressed

HDLC Sender[0] Set rollover count to ffffffff

OTX_HDLC_SENDER_EC_FRAME_COUNT_ZERO will synchronize the count.

HDLC Sender[0] Setting frame sent event TRUE

- The timestamp event is now enabled and the HDLC device will generate the OTX_HDLC_SENDER_EC_FRAME_SENT event when the HDLC packet has been processed. The command option ‘t’ can be used to send a HDLC packet. The following text should be displayed:

Key t pressed

Sending 1 HDLC frame(s) from HDLC Sender #0 (len=10)

HdlcSender[0] Event: HDLC frame completely sent

11-01 13:38:18.209   OTX_HDLC_SENDER_EC_FRAME_SENT frame count 39938, Frame Slot 0

g_coHdlcFramesSentRollOverCount count 0

11-01 13:38:18.214 HdlcReceiver[1]

HdlcReceiver[1] Len=10, Fe=0x00, Sq=48: 00 01 02 03 04 05 06 07 08 09

- The frame count in step 3 was 39938 with a rollover count of zero. This frame would have a timestamp of 39938 * 125microSec = 4.99225 Sec. In this example the rollover count was 0 but if it were any other value the rollover count times 8.192 seconds would need to be added to the frame count to determine the timestamp.

- The command option ‘+’ can be used to modify the number of HDLC packets sent.

Key + pressed

Set the number of consecutive frame to sent when 'm', 't', or 'T' are pressed (1)

3

Consecutive frame count is now 3

- If this value is set to 3 then the following is displayed when the ‘t’ option is used.

Key t pressed

Sending 3 HDLC frame(s) from HDLC Sender #0 (len=10)

HdlcSender[0] Event: HDLC frame completely sent

11-01 13:38:26.719   OTX_HDLC_SENDER_EC_FRAME_SENT frame count 42301, Frame Slot 0

g_coHdlcFramesSentRollOverCount count 1

HdlcSender[0] Event: HDLC frame completely sent

11-01 13:38:26.721   OTX_HDLC_SENDER_EC_FRAME_SENT frame count 42314, Frame Slot 0

g_coHdlcFramesSentRollOverCount count 1

HdlcSender[0] Event: HDLC frame completely sent

11-01 13:38:26.721   OTX_HDLC_SENDER_EC_FRAME_SENT frame count 42327, Frame Slot 0

g_coHdlcFramesSentRollOverCount count 1

11-01 13:38:26.724 HdlcReceiver[1]

HdlcReceiver[1] Len=10, Fe=0x00, Sq=50: 00 01 02 03 04 05 06 07 08 09

11-01 13:38:26.724 HdlcReceiver[1]

HdlcReceiver[1] Len=10, Fe=0x00, Sq=51: 00 01 02 03 04 05 06 07 08 09

11-01 13:38:26.724 HdlcReceiver[1]

HdlcReceiver[1] Len=10, Fe=0x00, Sq=52: 00 01 02 03 04 05 06 07 08 09

- In step 6 each of the HDLC packet finished processing 13 frames apart.

# Reference Documents

The following documents provide further detailed information related to the T2p5x4Hdlc demo program:

- Programmer’s Guide for OTX Hardware Driver (Odin document # 1412-1-SAA-1006-1)

- API Reference Guide for OTX Generic Hardware Driver (Odin document # 1211-1-SDA-1006-1)

- Installation Guide for OTX PCI Adapters (Odin  document number 1512-1-HCA-1001-1)

# Glossary

- OTX – Odin Telecom FrameworX

- Li – Line Interface device (T1/E1 device in this particular document)

- DSP – Digital Signal Processor

- SDK – Software Development Kit

- API – Application Programmer Interface

- CPU – Central Processing Unit. Refers to the host PC (laptop) in this document.

- EEPROM – Electrically Erasable Programmable Read Only Memory.

- FPGA – Field Programmable Gate Array.

- LED – Light Emitting Diode

- LS – Least Significant

- MS – Most Significant