Application Note for T2p5x4MfR2

# Background

T2p5x4MfR2 is a simple example console application demonstrating how to use some of the tone detection devices that are included with the OTX HW SDK to implement CAS MF R2 functionality. So what is CAS? What is MF? What is R2?

The online encyclopedia Wikipedia provides the following explanation of Channel Associated Signalling (CAS), MF and R2:

Channel Associated Signalling (CAS) signalling is a form of digital communication signalling. As with most telecommunication signalling methods, it uses routing information to direct the payload of voice or data to its destination. With CAS signalling, this routing information is encoded and transmitted in the same channel as the payload itself. This information can be transmitted in the same band (in-band signalling), or a separate band (out-of-band signalling) to the payload

R2 is a 1960s- and 1970s-era CAS signalling protocol used outside of the former Bell System to convey information along a telephone trunk between two telephone switches in order to establish a single telephone call along that trunk. R2 is the name given to two broad protocol groups: R2 line signalling and R2 register signalling.

R2 line signalling is a family of protocols that govern the resource acquisition and resource release related to a two-party telephone call attempt and, if successful, the establishment of a two-party telephone call. Although in the 1960s R2 line signalling was represented as electrical pulses on a two-wire or four-wire circuit, by the latter 1970s these analog electrical pulses also could be represented in digital form by a signalling DS0 in the trunk, which is normally channel 16 in an E1 trunk.

R2 register signalling is a family of protocols that govern the conveyance of addressing information during the addressing phase and how the call attempt turned out during the disposition phase. Although in the 1960s R2 register signalling was represented by electromechanical devices that could generate multi-frequency audio tones and by electromechanical devices that could detect those audio tones, by the latter 1970s these electromechanical registers also could be represented by digitized PCM audio in DS0 channels of an E1 other than the R2 line signalling DS0 in that E1.

R2 is not a single family of protocols. R2 line signalling is not a single protocol. R2 register signalling is not a single protocol. These points cannot be emphasized enough as this forms the most common misconceptions about R2. R2 signalling refers to a vast number of variants of R2 that all resemble each other to varying degrees. For most R2 variants, each such variant of R2 is promulgated by the PTT of each nation and/or by a telecommunications equipment manufacturer who addressed that national market with a nation-specific adaptation of one of their products.

Even the international standard R2 is merely yet another variant of R2 that is neither a superset nor a subset of any national variant of R2. Many people, including companies who attempt to enter the R2 market with telecommunications products, do not realize that international standard R2 is not necessarily a normative reference for any particular national variant of R2. The international standard R2 was first promulgated by CEPT in the 1970s and then later by ITU-T with only modest evolution. The international version of R2 is literally an inter-nation protocol for R2 signalling that crosses national boundaries. Conversely, each national variant of R2 is intra-nation.

One unifying aspect of R2 register signalling variants is that all audio tones are combinations of two frequencies. Each pair of two frequencies is called a multi-frequency tone, or MF tone. Each call attempt has an origination end and a termination end, both to the multi-span call as well as for each span within that multi-span call routing. The origination end is called the outgoing register in R2 parlance. The termination end is called the incoming register in R2 parlance. The outgoing register transmits forward MF tones to the incoming register. The incoming register transmits backward MF tones to the outgoing register. The outgoing register detects backward MF tones arriving from the incoming register. The incoming register detects forward MF tones arriving from the outgoing register. The outgoing register transmits the first signal, but from that point forward the outgoing register merely passively responds to the prompting driven by each signal transmitted by the incoming register. (In modern era proleptic vocabulary that retrofits nicely, the outgoing register's first push signal to the incoming register is similar to a peer-to-peer message, whereas the incoming register's pull signals that request information from the outgoing register are client-server, where the client is the incoming register and the server is the outgoing register.)

The spelling of signalling in R2 signalling is consistently signalling with a double ell instead of signaling with a single ell because R2 is an international protocol whose standardization documents are consistently written using British spelling. There are no R2 signaling (with a single ell) standards in the United States, so the use of United States' spelling conventions is inappropriate for a strictly international standard.

R2 is mnemonic for Region Two signalling to differentiate it from R1 signaling, the North American MF signaling. Here region number two was envisioned in the 1960s to be Europe and region number one was envisioned in the 1960s to be the Bell System in North America. In practice the term R1 is rarely used for North American MF signaling, except to contrast with its European contemporary, R2. As time as marched onward, use of R2 signalling spread beyond Europe to all regions of the globe, including Mexico on the North American continent. This spread is due largely to European telecommunications manufacturers selling their older equipment designs at a discount to developing countries while selling their higher-speed, higher-density post-R2 SS7 equipment at higher prices in more-industrialized countries. There is even sparse use of R2 in Canada, which is largely under the influence of Bell System standardization in North America.

The T2p5x4MfR2 application does not implement the full MFC R2 protocol for a particular national flavor. Instead it demonstrates how the OTX boards can be programmed to detect and send the MF tones that are associated with this protocol group.

The demo application provides a simple console interface which utilizes simple keystrokes as commands.

## Application Code Organization

The entire T2p5x4MfR2 example application is contained in a single ‘C’-language source file. The file contains a large number of functions used to demonstrate various functions but the code is mainly separated into four part. The following paragraphs will discuss these four parts in further detail: initialization of global resources, the configuration of physical resources, the configuration of logical resources, and resource control.

### Initialization of Global Resources

The application shares a number of common or global resources within the code.

The main interface with the user is the console interface. It is initialized using the OtxConsoleInit() command. This single interface is used by all the resources to communicate with the user and allow the user to determine code behavior.

The application provides the use a single log file. The file name is determined by the define DEMO_DEFAULT_LOG_FILE, but can also be modified using the command line argument /L.

Another important command line switch is the /T switch. This switch provides the type of OTX target board that the application should use.

This application can be used with the following boards:

- Thor-2-PCI-Express (OTX_DEVICE_THOR_2_PCI_EXPRESS)

- Thor-2-PCI-Plus (OTX_DEVICE_THOR_2_PCI_PLUS)

- Thor-4-PCI-Plus (OTX_DEVICE_THOR_4_PCI_PLUS)

- Thor-8-PCI-Plus (OTX_DEVICE_THOR_8_PCI_PLUS)

- Thor-2-PCMCIA-PRO-1-PCI (OTX_DEVICE_SLEIPNIR_1_PCI)

However, although the application is written to support this particular set of OTX boards, the application can easily be modified to support any OTX T1/E1 board with DSP resources.

The Thor board listed above needs to be populated with a Vidar-55x4-ASM or a Vidar-5x4-ASM-PRO board. The Vidar board hosts the DSPs, which hosts the logical tone detection devices.

The application uses a single event queue for all physical and logical resources. The event queue is used by the resources to inform the application of changes in resource state or resource activity that may need the application to perform a given task. The event queue is monitored by a separate thread started by a call to OsaThreadStart(). There can be multiple event queue and threads created to ensure that an event is not blocked while another event is being processed. This is especially true for devices that are supplying data and can not wait for a lower priority event to finish processing.

### Configuration of Physical Resources

Physical resources are provided by the hardware. The board types supported by this application each provide a unique set of resources that can not change dynamically. These resources are physical devices with different hardware requirements. The application performs the necessary configuration needed to allow the data transfer and support for the logical devices.

The application provides three functions to perform this task: DemoCreatePhysicalDevices(), DemoConfigureE1T1Devices(), and DemoEnablePhysicalDevices().

The DemoCreatePhysicalDevices() function determines which devices are available with each hardware type and assigns the appropriate handle to allow the device to communicate with the application. It also determines if each device is operating correctly.

The DemoConfigureE1T1Devices() function provides the interface configuration.

The DemoEnablePhysicalDevices() function enables the device once it has been configured and is ready to perform the necessary task.

Two T1 or E1 spans are configured. The T1/E1 Line Interface devices are referred to as Line Interface (Li) devices. All of the OTX device numbers use a zero-count terminology. The two Li devices are therefore called Li0 and Li1.

The configuration parameters for the two spans are set to hard-coded values in the DemoInitT1E1ConfigData() function. The spans can be connected to another T1 or E1 device, or looped with an external loop cable.

This application programs loads the OTX_DSP_SPM_OTXSPM2 program module into the DSPs. It has support for a variety of tone devices. 3 different detector types are used in this program:

- OTX_LDEVICE_TONE_MF_DETECTOR is used for the MF R2 Forward tones

- OTX_LDEVICE_TONE_DUAL_DETECTOR is used for the MF R2 Backwards tones

- OTX_LDEVICE_TONE_SINGLE_DETECTOR is used for MF R2 Supervisory tones

An OTX_LDEVICE_TONE_SINEWAVE_GENERATOR device is used to generate the outgoing tones.

The CCITT R2 Forward tones are defined as:

1:      (1380Hz/1500Hz)

2:      (1380Hz/1620Hz)

3:      (1500Hz/1620Hz)

4:      (1380Hz/1740Hz)

5:      (1500Hz/1740Hz)

6:      (1620Hz/1740Hz)

7:      (1380Hz/1860Hz)

8:      (1500Hz/1860Hz)

9:      (1620Hz/1860Hz)

10:      (1740Hz/1860Hz)

11:      (1380Hz/1980Hz)

12:      (1500Hz/1980Hz)

13:      (1620Hz/1980Hz)

14:      (1740Hz/1980Hz)

15:      (1860Hz/1980Hz)

The CCITT R2 MF Backward tones are defined as:

1:      (1140Hz/1020Hz)

2:      (1140Hz/900Hz)

3:      (1020Hz/900Hz)

4:      (1140Hz/780Hz)

5:      (1020Hz/780Hz)

6:      (900Hz/780Hz)

7:      (1140Hz/660Hz)

8:      (1020Hz/660Hz)

9:      (900Hz/660Hz)

10:      (780Hz/660Hz)

11:      (1140Hz/540Hz)

12:      (1020Hz/540Hz)

13:      (900Hz/540Hz)

14:      (780Hz/540Hz)

15:      (660Hz/540Hz)

When these tones are detected the detection devices will generate an event that the application will pick up through calls to OtxDrvGetEventData(). The particular digit detected is passed in the m_nParam member of the OtxEventDataS struct.

### Configuration of Logical Resources

The logical devices are differentiated from the physical devices by the fact the number of logical devices is not set. More of them can be requested by the user and allocated by the system if needed. Logical Devices can be classified into Control Device and Data Devices:

- Control Device:  A logical device performing a control function. Like Event Queue.

- Data Filter Device: A device implemented in Hardware, Firmware of Software which converts, processes, or manipulates serial data. Examples of data devices include Time-Space Switch, HDLC Controller, Tone Detector, and Tone Generator.

- Data Filter devices can be categorized into 3 basic types:

  - Data Source: Originates a Pipe or adds data to an existing Pipe. Like a Sinewave Tone Generator.

  - Data Sink: Terminates a Pipe or extracts data from a Pipe. Like an MF Detector.

  - Data Bridge: Cross-connects data from one Pipe to another or combines data from multiple Pipes into one Pipe. Like cross-connect in TSS.

The application provides four functions to support the logical device: DemoCreateLogicalDevices(), DemoConnectDevices(), DemoInitializeLogicalDevices(), and DemoEnableLogicalDevices().

The DemoCreateLogicalDevices is similar to the one used to create the physical device. It is used to assign a handle to each device to allow for communication with the device. It is also used to determine if the logical device has the necessary resource available. It should also be noted that multiple logical devices can be supported by the physical device but there is a limit to the number of logical devices a physical device can support.

The DemoConnectDevices performs the connection of the devices using the pipe device. A Pipe Device is a logical entity representing a serial bit-stream and a subset of a Highway with a data rate between 8 kbit/s and 2.048Mbit/s. A pipe is a characterized by its capacity (in 8 kbit/s increments). A pipe is connected to physical or logical devices and mapped to specific time-slots. This application uses pipes with a data rate of 64 kpbs, which corresponds to one timeslot of the T1/E1 span.

The DemoInitializeLogicalDevices is called once the device has been created and connected. It is used to initialize the logical device.

The DemoEnableLogicalDevices function enables the device once it has been configured and is ready to perform the necessary task.

This application creates 64 instances of each the devices listed in the chapter above. 32 devices are associated with Li0/DSP#0 and the other 32 devices are associated with Li1/DSP#1. The device numbers for these logical devices run from 0 through 63. Device 0 and device 32 are not enabled since they connect to TS0 of their respective spans, which is the synchronization timeslot in E1 (and the F-bit timeslot in T1).

It is also worth to notice that this application uses the one-to-many pipe connection feature of the load DSP program. A pipe representing an incoming timeslot is connected to the three different types of tone detection (MF, Dual tone, and Single tone) devices “in parallel”.  This means that all three devices will receive the sample data from that timeslot, and all three devices will run its detection algorithm on the sample data. Having the three detection devices connected in parallel could potentially trigger multiple detection events for the same sample data. However, the devices can be tuned towards stringer detection criteria using so called attributes. This demo application sets some of these attributes for the tone detection devices. The attribute values might be to be tuned to match a particular national flavor of R2.

The loaded DSP program only supports one instance of an MF device. Therefore we use the MF device for the forward tones and we use a generic Dual Tone device for the backwards MF tones.

The dual tone detector can detect a number tone pairs (two-frequency tones). The frequencies to detect are supplied as a parameter to the OtxToneDualDetectorInit() function. This device can handle 6 frequencies. When a certain dual tone (MF backwards tone in our case) is detected, an event is generated and the detected dual tone is coded according to the table below:

| Frequency | 540 Hz | 660 Hz | 780 Hz | 900 Hz | 1020 Hz | 1140 Hz |
| --- | --- | --- | --- | --- | --- | --- |
| 540 Hz |  | 1 | 2 | 4 | 7 | 11 |
| 660 Hz | 1 |  | 3 | 5 | 8 | 12 |
| 780 Hz | 2 | 3 |  | 6 | 9 | 13 |
| 900 Hz | 4 | 5 | 6 |  | 10 | 14 |
| 1020 Hz | 7 | 8 | 9 | 10 |  | 15 |
| 1140 Hz | 11 | 12 | 13 | 14 | 15 |  |

For example if the tone pair 540/660 Hz is detected, then the Dual tone detector will generate an event with m_nParam set to 1. The application will then map this to MF Backwards digit 15, and print 15 to the log file.

As already mentioned earlier, the Dual Tone detector device is highly configurable to be able to detect various tones of various amplitudes. A detailed description of how the Dual tone device can be tuned and configured can be found in the OTX HW API Help files.

### Resource Control

The application provides the user with a consol interface which allows the user to control the resources. The application also provides an event queue and monitoring thread to control and process data provided by the physical and logical devices. The majority of the processing is performed while in the thread.

## Clock Source

The T2p5x4MfR2 application uses the OTX_CLOCK_SOURCE_LOCAL_0 clock source, meaning that the board will synchronize its clock rate to the data rate of the incoming E1 signal of Li0. The clock source can be changed by modifying the OtxBrdSetClocks() call in the function main(). E.g if to become a clock master for the E1 spans, change the OtxBrdSetClocks() call to:

OtxBrdSetClocks(

g_hPhysDevs.hThorController0,

OTX_CLOCK_MODE_LOCAL,

OTX_CLOCK_SOURCE_INTERNAL

);

## T1/E1 Configuration

The T1/E1 line configuration is set in the DemoInitT1E1ConfigData() function.

In E1 mode the application configures the line interface in CRC4 Multiframe/HBD3 format.

In T1 mode the application configures the T1 line interface for ESF/B8ZS frame format.

The configuration can easily be modified to match any possible T1/E1 configuration by editing the settings of the struct members in the DemoInitT1E1ConfigData() function.

# Usage Examples

## E1 Usage Example for Thor-8-PCI-Plus

To run T2p5x4MfF2 in E1 mode using the Thor-8-PCI-Plus board, the following command line parameters could be used:

T2p5x4MfR2 /mt /tp /b0           (for board 0)

T2p5x4MfR2 /mt /tp /b1           (for board 1)

Assuming that Li0 is looped to Li1, you can send one R2 MF Forward tone from current dialer. The current dialer defaults to Li0:TS1

press ‘l’

You should then receive an event the MF detector device connected to Li1:TS1. You should receive the OTX_LDEVICE_TONE_MF_DETECTOR event which will be handled in the DemoPrintEvent() function, which prints:

MfDetector[Li1:TS1] detected MFR2 Forward digit 1

You can change the current dialer by pressing ‘n’ to increment the dialer device number by one, and ‘N’ to decrement the dialer device number by one.

Press ‘h’ to get a list of all possible key commands.

# Reference Documents

The following documents provide further detailed information related to the T2p5x4MfR2 demo program:

- Programmer’s Guide for OTX Hardware Driver (Odin document # 1412-1-SAA-1006-1)

- API Reference Guide for OTX Generic Hardware Driver (Odin document # 1211-1-SDA-1006-1)

- Installation Guide for OTX PCI Adapters (Odin  document number 1512-1-HCA-1001-1)

# Glossary

- OTX – Odin Telecom FrameworX

- Li – Line Interface device (T1/E1 device in this particular document)

- DSP – Digital Signal Processor

- SDK – Software Development Kit

- API – Application Programmer Interface

- CPU – Central Processing Unit. Refers to the host PC (laptop) in this document.

- EEPROM – Electrically Erasable Programmable Read Only Memory.

- FPGA – Field Programmable Gate Array.

- LED – Light Emitting Diode

- LS – Least Significant

- MS – Most Significant